AutoWX.EXE

(C) 1994,95 N5KOA

VER 6.x   06/95

=====

NOTE:

=====  If you are upgrading to a newer version of AutoWX, read

the "Compatibility" notes below.

AutoWX (Auto-Weather) is a PC compatible packet program that

accepts weather data from the Ultimeter II weather station and

makes the data available on packet by direct connect requests

and/or via automatic beacons (in APRS beacon format).  The

program assumes you are using two serial ports (COM1 and COM2)

and that the U-II is connected to one, the TNC to the other.

Use ALT-H for online help.  The program works very well on an XT

with 256K RAM and a single floppy disk drive.  XT's are often

GIVEN away these days and one could be dedicated full time to

this application.  (If you don't have the serial data cable for

the U-II, see TIPS below.)

Features:

Logs weather data in a binary log file that can be read

by the Data Viewer software sold by the U-II manufacturer

(Peet Bros.)

Split-screen display shows weather data, including APRS

weather beacons from other stations, for past 40 minutes

on bottom - packet on top.

Displays Hi/Lo Temperatures and Hi Wind with the time of

day.

Continuously reads U-II data port to show highest wind

speed for each 5 minute interval.

Allows packet stations to connect and request current

weather conditions, including rain, and highs and lows

for the date.

Sends weather beacons in APRS format.

Logs weather beacons from other AutoWX or APRS stations.

Creates ASCII text file showing highs and lows for the

date.

Serves as a simple packet terminal program.

Runs on a minimal computer system  (256K, monochrome,

floppy, 2 serial ports).

Thanks to the many users of AutoWX around the country who have

offered suggestions, comments and compliments!

Compatibility with earlier versions:

The WX.CFG file from earlier versions should be deleted and a

new configuration file created by running the new AutoWX.

The WX.DAT file from earlier versions should also be

deleted or edited to include the year-to-date rain total.

If you edit WX.DAT, add a comma and a value equal to ten

times the YTD rain amount.  (If the YTD rain amount is

11 inches, add ,110 at the end of the line.  Or ,1100 if

you are using the 100ths inches rain gauge.)

STARTUP / CONFIGURATION Parameters

This program is updated often with fixes and new features.  If

you have suggestions, please contact me by packet or U.S. mail.

---------------------------------------------------------------

= THE ULTIITSR PROGRAM SHOULD NOT BE USED WHILE AUTOWX IS RUNNING =

---------------------------------------------------------------

The TNC ECHO should be ON, AUTOLF OFF, CSTAMP ON and MAXUSERS 1.

If you want to send the CTEXT upon connect, turn CMSG ON, also.

If you are running a very slow 8088 machine, you may have to

turn MONITOR OFF.

The first time AutoWX is executed, or if the WX.CFG file is not

found, the user is prompted for configuration information.  The

next few paragraphs are in order as needed for configuration.

The COM port of the TNC must be identified as (COM) "1" or "2".

The other COM port will automatically be assigned to the

Ultimeter II.

Supported baud rates for the TNC port are: 1200,2400,4800 & 9600

Your station's latitude and longitude are necessary to beacon in

a format recognized by the Automatic Packet Routing System (APRS

by WB4APR). There is very little error validation in the entry

of the coordinates so be very careful that they are entered in

the format shown at the config prompt.  Null entries at the

Latitude and Longitude prompts will result in "0000.00N" and

"00000.00W".

A temperature adjustment value is included because the Ultimeter

II temperature sensor can be affected if additional lengths of

cable are attached to it.  The default value for the temperature

adjustment is 56 which will be correct if the display on the

U-II is correct.

If you are using a rain gauge, you may enter 10 or 100 to

indicate that the rain gauge measures in 10ths or 100ths of an

inch.  A null entry defaults to 10.

When prompted for the Log File Name, your entry should NOT

include an extension.  The extension .LOG is automatically

added.

After the configuration information is entered, it is written to

the ASCII file WX.CFG.

WX.DAT file

The WX.DAT file is an ASCII file that tracks the high and low

temperatures and high wind speed for the day.  If the file is

not present when AutoWX is executed, it will be created.  The

file is updated whenever the Hi's and Lo's are edited (ALT-E),

when the program is terminated with ALT-Q, and at 50 minutes

after the hour when the buffer is flushed.

The Hi's & Lo's can be edited by typing ALT-E during the

program.  At that point you are prompted to enter  1,2 or 3

depending upon which item you want to change.  If you enter no

value, the edit is aborted and the program continues.  If you

change a value, enter the new information in the same format as

displayed.  IMPORTANT:  While in the ALT-E Edit, the com buffer

remains open.  If the buffer overflows while you are in the edit

window, an error will occur.  Do not allow the edit window to

remain open very long.  To reset the wind chill data in the

WX.DAT file, edit the file with an ASCII text editor and set the

wind chill to 0 and the time to "" (null in quotes).

Alternatively, you can just delete the WX.DAT file, re-run

AutoWX and then use ALT-E to set hi's and lo's.

If you stop the program and restart it on the same date, the

data in the WX.DAT file will be read and used by the program.

If the date in WX.DAT is different than the system date on your

computer, the data will be ignored and new Hi's and Lo's will be

tracked beginning with the first data read on the wx port.  Hi's

and Lo's are reset at midnite.

UPDATE

An "update" occurs when the program is started, every five

minutes, and when a connected (//WX) request is received.  When

a timed update occurs (every 5 minutes), the data is logged to

disk.  The U-II data is read continuously and logged to disk

every 5 minutes.  Whenever the data is updated, it is also

printed on the lower portion of the screen.  Updates can be

toggled off with ALT-U but this will also disable the logging of

data to the log file.  If you are going to disconnect or

otherwise disable the weather input, toggle the updates off with

ALT-U to keep the program from hanging up during the update

sequence.  While the UPDATEs are toggled OFF, connected requests

for weather (//wx) will receive a message stating that "Weather

data is temporarily unavailable...".

When UPDATE is toggled off, the timer is not reset; when

UPDATE is toggled back ON, if the programmed time (5 min.) has

passed while UPDATE was OFF, the update will be performed

immediately.

WEATHER NODE (connected requests)

If another packet station connects and sends "//wx", (without

the quotes - upper or lower case), AutoWX will update and send

the current weather data and Hi's and Lo's for the day.

I include instructions in my Connect Text (CTEXT) showing:

"Send //wx for weather data - //bye to disconnect..."

(Don't forget to turn CMSG ON!)

Toggle the Wx Node off (ALT-X) while you type into CTEXT the

//WX and //BYE then toggle it back on.  Otherwise, AutoWX will

attempt to send the weather info when it sees "//WX" and will

send a disconnect sequence when you type "//BYE".  (You could

also use a different terminal program; the message is stored in

the tnc, not in AutoWX.)

While a station is connected, the weather Beacon is suspended.

Also, if UPDATES are OFF a connected user will see the message,

"Weather data temporarily not available...".

The last ten stations that connected can be viewed by pressing

the PgDn key. (You MUST have CSTAMP turned on to see the

stations list with PgDn.  AutoWX looks for brackets ([]) around

the date and time stamp to parse the users callsign.  If you are

using a tnc that does not conform to this format, let me know

and we'll try to support your specific tnc in the future.)

If you enter a "greeting" with ALT-G, the message will be sent

to connected users.  You might use this to notify users to check

the mailbox for bulletins, etc.  Remove the greeting message by

entering a null response to the ALT-G prompt.

Users may disconnect by sending //bye.

BEACONS

Automatic beacons can be toggled ON/OFF with ALT-B.  The beacon

will be digi'd through any valid path specified in the UNPROTO

tnc parameter. UNPROTO should be set to APRS or to APRS VIA xxxx

in order to be identified by stations running APRS.

When Beacon is toggled ON, the user is prompted for the interval

(in minutes;  default = 60 minutes) and for any text to be

tagged on the end of the weather data.  The Beacon time will be

in the nearest 5-minute increment regardless of the value

entered;  for example, if you enter 9 minutes, the actual beacon

time will be 10 minutes.  Beacons are transmitted only when the

log file is updated.

The beacon will automatically include the Longitude and Latitude

from the config file.   If you enter text to be appended to the

beacon (Btext), it will be limited to 30 characters.

The Beacon can be forced to xmit immediately by typing ALT-T.

After using ALT-T, the beacon timer (if Beacon is ON), restarts.

The Beacon can be turned off/on remotely with //BEACON.  Make

sure that you have the correct path specified in UNPROTO.  The

default time for beacons turned on remotely is 20 minutes.  You

can change this by starting AutoWX with the /RB switch.  You

will be prompted for the new interval.  The new time will remain

in effect until the program is stopped.

If you enable Weather Beacon Logging with ALT-W the program will

capture any monitored APRS-type weather beacons and save them to

an ASCII text file named WX.LOG.  While Weather Beacon Logging

is enabled, an asterisk (*) will appear to the left of the UPDATE

status at the bottom of the screen.  Weather Beacon Logging may

not be compatible with slower 8088 class computers and/or you may

have to use a slower baud rate to the tnc.

RAIN

To keep up with the latest APRS beacon format, rain amounts, in

tenths of an inch, are beaconed in the /R000 (hourly) and /P000

(daily) format.  The hourly amount is updated every five minutes

when the 5-minute update is done for the binary log file.  Note

that if AutoWX has not been running continuously for at least

one hour, the hourly rain total may be incorrect because the

hourly rain amount is calculated by AutoWX, not by the U-II.

The binary log file continues to log rain in Upper and Lower

totals just as done by the ULTIITSR.COM program that comes with

the Peet Bros. software.  (Beacons show rain in 10ths inches

even if you have the 100ths rain gauge.  100ths are rounded up

to 1/10th at 7/100ths for beacons only.)

Updates on the screen show the amount of rain for the past 60

minutes along with the amount of rain for the day in the format:

(hour ->)  0.00 / 0.00  (<- day)

Connected weather requests (//WX) show hourly, daily, monthly

and year-to-date rainfall.  The monthly rain total indicates the

Upper Rain total from the U-II.) The Year-To-Date rainfall

amount is actually the value recorded in the WX.DAT file plus

the U-II lower rain amount.  The YTD amount in the WX.DAT file

can be edited by using ALT-E just as you do to edit the temps,

etc. Edit the YTD amount whenever you clear the U-II lower rain

total to keep it accurate.

LOG File

Every 5 minutes the program updates the binary logfile named in

the WX.CFG file.  This file can be read with the Ultimeter II

Data Display program.

AutoWX creates a new log file at 0000 HRS on the first of the

month. The older log file is named MMYY.LOG where MM = month and

YY = year.  A simple file-reading program is also available from

me... see TIPS at the end of this document.   (Log files can be

appended to each other by using the COPY /B command.  Example:

copy /b file1.log+file2.log bigfile.log  See your DOS manual for

details on COPYing binary files.)

NOTE:  Logging is suspended while UPDATE is toggled OFF.

The U-II Data Display program assumes that all log entries are

at five minute intervals.  Therefore, if you suspend logging for

more than a couple of minutes, the Data Display program will be

reading data that was logged after the time displayed in the

Data Display program.  For example:  If you suspended the logger

with ALT-U at 10:28 and did not resume logging until 10:58, the

Display program will show the data logged at 11:00 as if it were

logged at 10:30 and there will be thirty minutes of no data at

the end of the day.  See Log File Notes for more information.

TERMINAL Mode

If you use the program as a packet terminal to connect to other

stations temporarily, turn the Beacon OFF (ALT-B).  If UPDATEs

are left ON, the logger will continue to log data and the lower

half of the screen screen will continue to receive updated data

every five minutes. The program will ignore the //wx request if

you disable it with the ALT-X toggle.

TIME SET (ALT-Z)

If you need to reset the system clock because it runs slow or

fast, type ALT-Z and enter the new time in the format HHMMSS.

For example, to set the time to 14:34:15, enter 143415 without

the colons.  This feature does no error-checking so use it

carefully.  The new system time is re-displayed immediately

after you enter a time.  If you press ENTER without entering a

new time, the old time remains intact.  IF YOU ENTER A TIME

EARLIER THAN THE LAST UPDATE, THE PROGRAM MAY NOT UPDATE FOR AT

LEAST 24-HOURS OR UNTIL YOU RESTART THE PROGRAM.

There are several utilities available to "automatically" adjust

your computer's clock if it requires frequent adjustment.  Check

out CLOCKWORK (CWK.EXE) by Pavel Otavsky or the PC MAGAZINE

utility, WTIME.

=====  Always double check the time if you change it!

NOTES:

Log File Notes

The binary log file is much like the one created by the

Ultimeter-II Data Logger and is very efficient.  You can

probably get a whole year's data on a 360K floppy.  The AutoWX

program must run for at least ten minutes before any useable

data is logged.  Also, starting and stopping the AutoWX program

before any useable data is logged can cause errors in the

display.  The U-II Data Display program reads a file header in

the log file to determine the starting date and time but all

data entries between headers are assumed to be in five minute

intervals.  The AutoWX program writes a new date/time header

each time it starts but if logging is suspended by the ALT-U

toggle, the data may miss the 5 minute interval and therefore be

incorrectly displayed by the U-II Data Display program. If the

logger is suspended for more than 10 minutes, quit the AutoWX

program and restart it to avoid the problem.  Although data may

be missing for the suspended period, the data logged will be

displayed with the correct time if you follow this procedure.

The AutoWX program reads the Ultimeter II port continuously and,

when logging, logs the highest windspeed recorded during the

last 5 minutes.  However, the U-II data sends serial data every

4 seconds (on average) but reads sensors about every 1.5 seconds

and there may be differences in the high windspeed recorded on

the U-II display and that recorded by AutoWX.

AutoWX is released for the use by other licensed amateur radio

operators and no payment or registration fees are necessary.

The program is copyright protected and I choose to retain all

rights to the code.  The source code (QuickBasic 4.5) will not

be made available unless somebody convinces me they can

substantially contribute to the project.  AutoWX may NOT be

distributed with any commercial merchandise without written

permission from the author.

Comments and questions can be directed to N5KOA at N5RKN.#WTX

(Odessa, TX).  Please drop me a line if you try the program.

Disk copies are available only when accompanied with enough

green stuff to pay for the postage, disk, etc. DO NOT SEND

DISKETTES!  ($5 or more gets you some utility programs to go

with AutoWX; like WxRead.EXE for reading/converting the binary

log file.)

Ultimeter II weather station sources:

PEET BROS. as advertised in QST

Radio Com Sales  (Larry, N5VFW)

Box 118814

Carrollton, TX  75011

(214) 492-3824

APRS is a map-based packet program by WB4APR, Bob Bruninga,

Annapolis, MD

WXSTAT.EXE - a program to monitor Ultimeter II weather data in

realtime.

PROGRAM UPATES

Ver. 6.x  - CURRENT

6.1x - Record U-II lower rain amount to WX.HST file at

midnite

Errors/Bugs and other annoyances...

If you start AutoWX WITH A FLOPPY DRIVE, make sure the startup

floppy is in it's drive at 50 minutes after the hour.  The write

buffer is flushed at that time and the disk must be found for

the program to continue. Also, do not attempt to use the ALT-C

copy feature if the copy will attempt to copy files onto

themselves.

HF transmissions will often interfere with the U-II and bad data

can be passed to the AutoWX program.  The use of an rf filter is

highly recommended.  (A 30 db filter is available from PEET

BROS.  Also, I've seen telephone RF filters at WalMart.  You may

need more than one.)  AutoWX attempts to detect and discard invalid

data before it is displayed or logged.

Data for the binary log file is logged every five minutes but

data is not physically written to disk until the write buffer is

full (one disk sector) or until the program flushes the buffer.

If power to the computer is interrupted, any data in the buffer

is lost.  As the program writes only 4 bytes (on average) at a

time, 10 hours or more could be lost.  This problem is

characteristic of DOS I/O and a more suitable solution has not

been determined yet.  To write the data every five minutes would

result in, what I feel is, excessive drive wear.  I will

continue to look for alternatives. In the meantime, the buffer

will flush every hour at 50 minutes after the hour.

The National Weather Service calculates wind speed using a

2-minute average of sustained wind.  AutoWX records the highest

wind speed during a 5-minute period.  The HI WindSpeed indicated

by AutoWX is closer to a "gust" windspeed if you want to be

technical.

It has been reported that there are two versions of the U-II in

regard to serial data output.  My U-II sends an asterisk (*) at

the beginning of each data stream and there is apparently a

version that sends the pound sign (#).  The (*) is supposed to

indicate wind speed in MPH while the (#) indicates KPH.  If your

U-II sends in KPH (#), start the AutoWX program with the switch

/KPH.  Example:  AUTOWX /KPH

If you find a bug in the program, check for a later version on

the ARRL BBS (203) 666-0578.  If the bug exists in the newest

version please let me know so I can fix it.  I also welcome any

suggestions or comments so please let me know if you like or

dislike the program or if you would like to see other features

added.  I will also consider customizing your version if you

will pay for the disk and postage.

Les Blalock, N5KOA

1711 Pagewood

Odessa, TX  79761

(915) 332-7112 day

332-1942 fax

367-0408 eve  after 6 pm CST

TIPS

If you build your own data cable, add an rf choke on the data

lines in the DB25 housing.  (Looking at the 4-wire modular plug,

latch away from you, wire down, the third wire from the left is

the data wire which goes to pin 3 (DB25, pin 2 on DB9) with the

5K resistor connecting pins 3 & 2).   The first wire on the left

is the signal ground (pin 7 DB25 - pin 5 DB9).

If you use the TNC Command that inhibits the display of incoming

characters while you type (FLOW), turn it off.  Better yet, make

sure you hit the ENTER key the last thing after using your

keyboard. After hitting ENTER the cmd: prompt should appear on

the screen to let you know that there are no characters in the

tnc buffer.

Turn off your computer monitor and leave it off when not sitting

at the keyboard.  (The screen will burn in the image of the

weather data over time.)  My monitor stays off for days at a

time although I verify everything's running fine by copying the

data file to floppy and looking at it every day.

Import WX.HST file entries to a database program and create

custom statistical reports and graphs.  The WX.HST file is easy

to import as a "comma-delimited ASCII file" into most popular

database programs and many spreadsheet programs.

If a BBS in your town supports modem connections, consider

leaving your weather node on the BBS frequency (or a gateway

port) so the node can be accessed by telephone, too.

An ASCII error log file (WX.ERR) is created if an error is

generated by AutoWX. If this file appears in your directory you

can look at and/or delete it. Just make sure it doesn't get big

enough to eat up disk space.

A program to read the binary log file and write it to an ASCII

file (WXREAD.EXE) is available for $5 (postage and diskette

included).  Send requests to the address above.

If the Upper RAIN total is zeroed, the program will continue to

log the previous URAIN amount until midnite or until the program

is restarted.

The algorithm used to calculate wind chill is:

Wind Chill = .0817 * (3.71 * SQR(v) + 5.81 - (.25 * v) *

(t-91.4) + 91.4

t = ambient temperature in degrees F

v = wind speed in miles-per-hour

I have just found some information on the MPX2100AP 15-PSIA

pressure sensor (Motorola) and would like to use it to build a

barometer that could be interfaced with AutoWX.  It seems that

it would be fairly easy to interface to a game card.  Just

think, with barometric pressure, AutoWX could provide basic

forecasts!  If somebody beats me to this project, let me know so

we can work together to include it in AutoWX.

If you need to re-set your U-II rain amounts (or if you just

want to play with the rain amounts), attach a momentary switch

to a 4-wire telephone cable with a modular plug and attach it to

the U-II junction box in place of the rain gauge cable.  By

shorting two of the wires you can cause the rain display to

increment so if you put the switch on the right two wires you

can make it "rain" all you want.  I don't remember which two

wires are used because it's been so long since I made mine - but

it won't take you long to find the right ones.  Remember though,

the display will not register the "clicks" very quickly - allow

5-10 seconds between clicks.

If AutoWX is started with a /Q switch, no header will be written

and the next log entry will not occur until the computer time is

xx:x0 or xx:x5.  This is useful if you exit AutoWX for only a

minute or so and then restart AutoWX.    REMEMBER though, no

header is written so the next entry in the log will be assumed

to be five minutes after the last logged entry.  Use this

function with care to keep your log file accurate.

AutoWX Quick Guide

ESC   Clear Screen

PgDn   Show last 10 connected stations

ALT - B   BEACON on/off

ALT - C   Copy data files to Floppy

ALT - D   Shell to DOS

ALT - E   Edit/See WX.DAT

ALT - G   Enter GREETING message for additional "connect text"

ALT - Q   Quit

ALT - T   Xmit Beacon

ALT - U   UPDATES off/on

ALT - W   Wx Beacon logging  (shows * near UPDATE when on)

ALT - X   Wx Node off/on

ALT - Z   Check/Set TIME clock

Startup Options:

/B:   Copy data files to DRIVE B: instead of DRIVE A: when

using ALT-C

/KPH   Assume Ultimeter II output is in kilometers per hour

/RB   Change remote beacon time    (default = 30)

Files:

WX.CFG  AutoWX configuration data.  Includes:

TNC port                 TNC baud rate

Latitude                 Longitude

Temp adjustment          Rain gauge meas.

Logfile filename

WX.DAT  Data file containing current Hi's/Lo's for the date.

Date             Wind Speed               Wind Time

Wind Direction   Hi temp                  Hi temp Time

Lo temp          Lo temp Time             Chill temp

Chill temp Time  YTD Rain amt.

WX.HST  Data file showing Hi's/Lo's for each date

WX.ERR  Data file created only if program generates an error.

(This file can be deleted at any time.)

ERROR CODES LOGGED TO WX.ERR:

#5  - Illegal function call      #64 - Bad file name

#11 - Division by zero           #68 - Device unavailable

#24 - Device timeout             #69 - Communications-buffer overflow

#55 - File already open          #71 - Disk not ready

#57 - Device I/O error           #72 - Disk media error

#61 - Disk full                  #75 - Path/File access error

#62 - Input past end of file

WX.LOG  Data file of any APRS beacons recorded from other

stations.  (MONITOR must be on)