AX25 driver for BAYCOM-like modem.

Beta version, 15th of Jan 1993

Free license is granted for _amateur_ use only.

AX25.COM is a packet driver comforming (to some extend) to well known

"FTP packet driver" specification. It's purpose is to serve as interface

between application software (e.g. KA9Q NOS) and a modem connected

to RS232 port (e.g. BAYCOM modem).

The driver relies on the following RS232 pins:

RTS - controls PTT: high level enables the transmitter

DTR - transmitted data. On this pin the driver sends data

to be transmitted by the modem.

When in receive state this pin is held high.

CTS - received data. The modem should supply here the data it receives.

DCD - the modem may supply here "carrier detect" signal.

This pin is _optional_ - it is not a must because the driver

is able to deliver carrier signal from received data.

GND - a common ground for the above signals.

TxD outputs a signal which is in high (positive voltage)

most of the time. A low power modem (TCM3105) can be supplied

from RTS+DTR+TxD (adding realized with diodes).

My primary intention while writting this driver was to provide

opportunity to run KA9Q NOS with simple modems based on TCM3105

or AM7910 chip. I tested the driver _only_ with NOS - no other software

was available at the time. Thus I can not guarantee full compatybility

with "FTP packet driver" definition.

The driver status can be examined with PKSTAT.COM and terminated

(removed from memory) by TERMIN.COM. These two utilities are part

of packet drivers package from Clarkson.

AX25 driver heavily relies on quick interrupt response.

Thus the application software should avoid disabling

CPU interrupts for long periods. Or say it in another words:

enable interrupts whenever possible.

AX25.COM has several start-time options. To see them start it

with "-?". If you start the driver without any command line

switch it will use default values: COM1, 1200 baud, etc.

These are the options - default values are in []:

-b<bit rate> [1200] - decimal number

defines the bit rate. Rates below 300 are not accepted

because of PC's timer specific features.

Maximum usable rate is probably 2400 but it really depends

on your CPU... The upper limit for this parameter is 14400.

"Valid" rates are only numbers which can be expressed as 14400/n.

If you give an "invalid" value it will be adjusted to the closest

"valid" one.

-i<software interrupt> [60] - hexadecimal number

Tells the driver which software interrupt to use

for control functions. Usually you would use numbers

from 60..63 here because these are intended for user applications.

-B<COM port I/O base> [3f8] - hexadecimal number

I/O address of RS232 port: 3f8 for COM1, 2f8 for COM2

-I<COM port IRQ> [4] - hexadecimal number

Interrupt request line of RS232 port: 4 for COM1, 3 for COM2

Only the range 2..7 is supported so far. I'm not sure the irq 2

would work on a PC/AT.

-s<slot time> [120] - decimal number

Slot time for p-persistance scheme. The value is in data bits.

For 1200 baud, slot time 120 bits means 100 ms.

-p<persistance> [64] - decimal number 0..255

p-persistance - the higher the value the higher the probability

of activating PTT at each time slot.

Your station becomes more "agressive" on the air

with increasing persistance and decreasing slot time.

-h<tx header length> [480] - decimal number 8..65535

Number of extra bits the transmitter sends before actuall data

is transmitted. 480 bits means 400 ms at 1200 bps.

You should always make this number as small as possible

for best bandwidth use. 480 tells you basicaly that 480 "useless"

bits (60 bytes) are being transmitted before real packet data goes in.

Unlike most TNCs this driver sends a square wave not a series

of HDLC flags in front of a packet. This was easier to program

in software and it makes DPLL lock faster at receiving end.

Thus less header bits should be needed...

-t<tx tail length> [24] - decimal number 8..65535

Number of extra bits the transmitter sends _after_ the actuall data.

24 bits makes 20 ms tx tail.

-c<carrier sense mode> [t] - one of the following letters: f,c,t,d

mode      meaning

f      full duplex - transmit whenever data is pending.

This option totally disables carrier sensing.

c      sense modem DCD line to find out whether channel is busy or not.

Your modem has to supply this signal. Note that TCM3105

does not do it very well... it sets DCD to high on any noise

on audio input.

t      sense data transition - if incoming data signal moves

the driver assumes that the channel is busy - BAYCOM uses

the same (or very similar) way up to version 1.4

d      deliver "channel busy" status by analysing incoming data.

BAYCOM v1.5 does effectively similar thing although

it uses different algorithm.

With this mode you may run your radio with squelch open

all the time.

How this option works may depend on modem type. Some modems

have still very regular digital output signal even with white

noise applied to the analog input.

Please note that argument line is case sensitive and so are hexadecimal

values... "2F8" will not do - you must specify "2f8".

This release features major transmiter code rewrite to make

its timing more accurate. By the way transmitter buffer

is being used with greater efficiancy.

Version from April 27th contained a bug in tx buffer handling

- after transmitting about 7 KB of data the transmitter

got stuck forever. This bug is now fixed.

Getting started quickly:

1. Start the ax25.com - most important parameters are COM base and irq.

e.g. for COM1: ax25 -B3f8 -I4

2. Start KA9Q NOS.

type in:

attach packet 0x60 ax25 5 1500

trace ax25 111

you should see packets being received now...

3. Try to connect to another station by typing:

connect ax25 <callsign>

If you realized that you started ax25.com with not the parameters

you liked use termin.com to terminate the driver (e.g. termin 0x60)

and start ax25.com again with another option set.

How big packets can ax25 driver handle ?

The receiver can handle 2048 byte frames.

This include address, control and data field but not CRC.

The transmitter buffer can hold up to about 8 KB.

That is the amount of data which can be transmitted

per each PTT push.

How to connect a modem to RS232 port ?

To cooperate with ax25 driver the modem should meet some

minimal requirements. Basic roules are:

1. The modem must provide decoded received data on CTS pin.

2. The driver outputs data to be transmitted on DTR pin.

Modem should modulate and pass this signal to the radio.

3. Pin RTS controls the PTT of the radio. When it becomes positive

the radio (together with the modem) should go into transmition mode.

4. Optionally the modem may provide carrier detect signal

on DCD pin.

5. Don't forget about GND line with is the reference for all above signals

The problem with most modems is level conversion... RS232 ports

of a PC use +/- 12 V while most modem chips need TTl levels (0..+5V).

The simple (but not very elegant) way of doing conversion is to use

CMOS inverters (I use 4049 with my TCM3105). Like on the picture below:

_____    |\

RS232 DTR   ---|_____|---| \o_____ TTL modem TxD

50 or 100 k | /

|/

_____     /|

RS232 CTS   ---|_____|--o/ |_______ TTL modem RxD

2.2 k    \ |

\|

_____     /|     /|

RS232 DCD   ---|_____|--o/ |___o/ |____ TTL modem DCD

2.2 k    \ |    \ |                      Modem Tx control

\|      \         ____+5V__      |

|   _|_      |        Radio PTT

Si diode  _|_  | | 3M   |          |

/_\  |_|      |          |

_____    |\     _____  | |    |    |  |\   |  ___ B |/  C

RS232 RTS   ---|_____|---| \o--|_____|-| |----|----|--| \o---|___|--|

50 or 100 k | /    2.2 k  | |            | /    10 k   |\

|/            10 uF          |/              | E

-------

RS232 GND ---o

|

_|______ 0V

Inverters are supplied from 0 and +5 Volts - same levels

as modem TTL part.

The first scheme _does_ work because CMOS inputs accept voltages outside

power supply range thanks to input protective diodes.

The resistor limits the current flowing through these diodes.

You must _not_ avoid it !

The second scheme does work as well because the threshold between

logical states in PC's RS232 is a bit _above_ 0 Volts.

BAYCOM team recomends HC or HCT series circuits - they must have

a reason for it but I don't know what it is...

Please note that transmitted and received data polarity is not important

in packet radio. Only transitions matters. However polarity of DCD signal

_does_ matter.

The last scheme shows PPT circuit with simple watchdog timer. It limits

the transmition time and is recomended for unattended stations

in case of software failure.

It is a good practice to ground unused RS232 inputs (RI, DSR, DCD).

Due to capacitive coupling to neibour pins there may apear short spikes

on them - these will trigger extra interrupts thus loading the CPU.

Have fun and _please_ send success/failure notes

and comments/hints/complaints to:

jalocha@chopin.ifj.edu.pl

or  jalocha@vxcern.cern.ch

or  jalocha@priam.cern.ch

my home address:   Pawel Jalocha

Rynek Kleparski 14/4a

PL-31150 KRAKOW (Poland)