FODTRACK Satellite Tracking System

==================================

Version 2.0

User Manual

===========

Introduction:

=============

FodTrack is a simple, straightforward program intended to control an azimuth-

elevation rotator like the Yaesu-Kenpro 5400/5600, and a transceiver, for any

kind of satellite orbits.

FodTrack runs in foreground, continuously displays what it is doing, uses no

special system resources, and is so simple to use that you will not have any

trouble trying to figure out what is going wrong when the silly satellites seem

to go in another direction than your antenna...

The rotator can be connected to any parallel port using an interface built

according to the schematic provided with the program. LPT2 is a good candidate, as

it is often present and seldom used. It can also be controlled via any serial

port, using the Yaesu GS-232 interface. Nonstandard ports are welcome, just

you need to know the address. No IRQ is needed.

The radio can be connected to any serial port. Just the same as for the

rotator port, only the address needs to be known to be able to use any port.

FodTrack is best suited for those setups where a single PC runs a multitasking

environment, like DESQview, with the satellite software in one window, a BBS in

another, and your favorite game in the third. Just open a fourth window and put

FodTrack there, it will be happy. It works with WISP under Windows too, but it

has not been specially written as a Windows-based program.

You can also use an independent computer for FodTrack. But you cannot run it in

background. If you want background operation, there are other programs

available for you.

FodTrack implements antenna flipping, so the satellites will not run against

your rotator's end stop. Rotator stop position can be south or north.

The FodTrack rotator interface cannot drive the rotator against its stop

position. The limits are set in hardware, so even if your computer goes crazy,

locks up, or whatever, your rotator will not be damaged by being driven against

a mechanical end stop.

It can run under manual or automatic control. In automatic mode, a skeduler

like WA2N's SatSked can provide the commands. Two modes for automatization

are provided: In the preferred one, FodTrack runs continuously and is

controlled via a command file; in the alternative one, FodTrack is called for

a specific satellite, and aborts after one pass.

You can use FodTrack to control only your rotator, only your radio, or both.

Control for Yaesu FT736, Kenwood TS790 and several Icom radios is implemented.

IK3NWV tested it on a TS790, while IK0XBQ made the Icom test with an IC820.

Thanks to them for the beta testing. On the FT736 I tested it, and I'm using it

permanently with that radio.

Probably it will run with other Kenwood radios, and perhaps also with other

Yaesu radios, but this has not yet been tested.

Copyright:

==========

FodTrack is free for noncommercial use. If you want to reward me somehow, write

a piece of useful software and put it in the public domain!

Disclaimer:

===========

FodTrack is provided without any guarantee that it will really do anything of

all the nice stuff this document says. But please, if you find a bug, tell me,

so I can fix it for the next version.

Error handling is not yet very complete. Be careful. In many situations the

program is case-sensitive.

Setting it up:

==============

This is VERY easy. Copy the FODTRACK files to a directory of your choice. You

can also maintain the kepfile there, but if you prefer you can use a kepfile at

some other place.

Edit the FODTRACK.CFG file to reflect conditions at your station. The file

explains itself.

Edit the FODTRACK.FRC file according to the satellites and frequencies you want

to use. Do not include those birds for which you don't want automatic

transceiver control. It's good enough to put the nominal frequencies into this

file. Later you can fine-tune the frequencies from inside the program.

You can define a "pseudo-satellite" in this file, called PARK. This will send

your Icom or Kenwood radios to the parking frequencies and modes specified

there. Yaesu radios return to the frequency they were on before the start of

the pass, regardless of any PARK data specified here.

If you will run the program from somewhere else, you need a path to the

FodTrack directory.

The kepfile must be in 2-line format, with the satellite name appearing above

each 2-line block. Title lines are no problem. They are not needed, but they do

not disturb.

Running it:

===========

Execute FODTRACK.EXE. The program will come up, read its configuration file,

and then it will read the command file (FODTRACK.CMD). The default command file

says "NONE", so the program will stay idling. That's a good time to look around

the screen:

You will see a status display, which says what the program is doing. It can be

idling, waiting for a satellite, tracking it, calculating AOS and LOS while

guessing if it is convenient to flip the antenna over, calibrating the

rotator, or accepting your incremental tuning input.

There is a nice clock, ticking away your valuable seconds; also there is a line

telling you that the satellite selection is automatic, and several windows

without any data in them. They will come to life when you select a satellite.

There's also a small reminder for the commands you have available. Please note

that four of these commands are only available while in manual mode.

During AOS-LOS calculation the keyboard is dead.

Now let's play a bit: Type the letter m to get the program into manual mode,

then t to start tracking a satellite. The program will ask you which satellite

you want. Enter it's name exactly as it appears in the kepfile, otherwise

FodTrack will be very unhappy with you. For example, type KO-23.

The program reads the kepfile, and tells you the age of KO-23's keps in the

proper window. It will also tell you if you can get good tracking precision

with those keps, or if you should get new ones.

Then it calculates AOS and LOS times (for the novice: AOS means Acquisition

Of Signal and LOS is Loss Of Signal) If you haven't disabled flipping, FodTrack

also looks into it's crystal ball, to see if the bird will run against your

rotator stop on the next pass. If so, it will tell you after a while that the

antenna will be "flipped". If not, it will be on the "normal" side. If you

chose to park your antennas in the configuration file, then the "Flipped" or

"Normal" display will be overrun by the word "Parked".

The AOS, LOS and flipping calculation takes only a few seconds if you have a

coprocessor, but it can take nearly a minute on slow non-coprocessor machines.

After flipping determination is complete, the program will start tracking the

bird. Every second, if the computer is fast enough, the position is updated.

If the AOS time is more than 2 minutes away, no data is sent to the rotator

nor to the radio.

When the great moment arrives, two minutes before AOS, the program will start

sending target position data to your rotator, indicating so in the rotator

status display. The two-minute allowance assures that the rotator has enough

time left to point at the satellite before it comes over the horizon. At this

time, the program will also start controlling your radio, and showing the

Doppler-corrected frequencies. If the FODTRACK.FRC file does not contain data

for the selected satellite, then the program will not access the radio, and

will display Doppler correction in PPM (parts per million). This is useful for

manual tuning of satellites.

If the rotator is not flipped, azimuth and elevation on the rotator are the

real ones. If the program had to flip the antennas, azimuth is 180 degrees

shifted and elevation starts backwards from 180 degrees.

If you selected a stepsize of zero degrees in the configuration file, then the

program will send the rotator position to the interface at a rate of up to once

every second. In this case, the rotator will move in fine steps, their size

being given by the dead-band in the interface and the stepsize of the D/A

converter (256 steps). If you selected a bigger stepsize, FodTrack will freeze

the rotator until the position error is half as big as your selected stepsize.

Then it will move the rotator a full step in the proper direction. If your

antennas have fairly broad lobes, you can use this feature to reduce wear and

noise. The recommended stepsize is about half of your antenna beamwidth.

When the satellite goes below the horizon, the rotator output is frozen, the

program stops controlling the radio, and calculates flipping for the next pass,

then starts waiting for the bird to come up again.

If you selected parking, the antennas will then be parked. Otherwise, they will

stay in the position the satellite left them, until the next pass is about to

start.

If you have an Icom or Kenwood radio, and you specified parking frequencies and

modes in the FODTRACK.FRC file, then your radio will be parked too. Yaesu

radios will return to whatever frequency they were on before the start of the

pass, if you leave the SAT switch in OFF position.

At any time except during AOS-LOS calculation, you can stop tracking a bird

using the s command, or quit using q. If you press a, the program will go back

into automatic mode, reading the command file and doing whatever it tells.

Tuning:

=======

While you are tracking a satellite in manual mode, you can enter the manual

incremental tuning mode. FodTrack will use the bottom bar on the screen to

display the theoretical frequencies at the satellite, the tuning mode selected,

and a list of available commands.

You can use the cursor keys to increment and decrement the frequencies. The

up/down keys change the frequencies in large steps, while the left/right keys

do the fine tuning. The size of these steps is definable in the FODTRACK.CFG

file.

There are four tuning modes: In RX mode, the receiving frequency is tuned. In

TX, it's the transmission frequency (sounds obvious, doesn't it...?). In DIRECT

mode both frequencies are tuned in step, which is useful for operating linear

transponders that don't invert the passband. Finally, INVERSE mode tunes the

RX and TX frequencies in opposing sense, for inverting transponder operation

like that on AO-10, AO-13 and FO-20.

When entering tuning mode, it defaults to RX tuning.

The SAVE command saves the modified frequencies into the FODTRACK.FRC file.

The EXIT command exits the tuning mode.

During manual incremental tuning the orbital calculation and everything else

continues. The theoretical frequencies are changed as you touch the proper

keys. Every second, the Doppler shift affecting these frequencies is

calculated, and while the sat is above the horizon, the corrected frequencies

are sent to the radio. So although the theoretical sat frequencies can be tuned

quickly, the frequencies in the radio are updated only once a second. This is

done because most radios don't support a much quicker update rate through their

control ports.

When you are first setting up FodTrack, you will need to fine tune the

frequencies, specially the RX frequencies of the PSK sats. You can tune them

until the data is being properly decoded and the signal is in the center of the

modem's working range, then SAVE them. You should not need to retouch this

tuning again for considerable time. Do this adjustment while using reasonably

fresh keps, the PC clock accurate to the second, and preferably near the start

or the end of pass, which is when the Doppler shift variation speed is minimal.

This same kind of tuning can be done for the FSK sats, using the FM

discriminator center meter, but tuning is far less critical on FSK.

The manual incremental tuning mode is also extremely useful on the analog

satellites. You can tune the RX frequency to some clear spot, then tune

your TX to the corresponding uplink frequency, then fine-tune it while

transmitting and receiving your own echo. After doing this, you can save the

frequencies, and from then on always use the INVERSE or DIRECT tuning modes.

Reagardless of Doppler shift and position inside the sat's passband, your

uplink and downlink will now always agree!  This allows for completely

hands-free operation, even on fast-drifting sats like FO-20... provided the

station you are talking too is also keeping his frequency on the sat constant!

The Kenwood TS790 is notorious for an unwelcome "feature": It will shortly mute

the receiver everytime it is commanded to update the frequency! So every

frequency update causes one or several packets to be lost. The reduce the

impact of this, FodTrack can be configured to send a frequency update only

when the frequency error is more than a specified value. But the only real

solution for this problem is to modify the radio, eliminating this muting.

The FodTrack Rotator Interface:

===============================

The program generates target position data as 8 bits for each azimuth and

elevation, using one of the printer control lines (pin 14) to tell if azimuth

or elevation is meant. 0 is az, 1 is el.

The schematic diagram of the interface is provided as a PCX file. This format

was chosen because in this particular case it gave the smallest file size among

all common formats, when compressed with PKZIP. The image is about 1700 x 1100

pixels; this seems to be the best compromise between file size and quality.

Please refer to the schematic in FODTRK22.PCX for this explanation:

A dual D/A converter (TLC7528 or similiar) is directly connected to the parallel

port. The port's strobe output is wired to the D/A's chip select input, pin 14

to the output select pin, and the 8 data bits to inputs 0 to 7.

The D/A converter is used in a reversed fashion, with the reference applied to

the outputs, and the output taken from the reference terminals. This allows

using a low-cost current-mode DAC in voltage-mode.

The analog outputs are compared to the rotator position output using

differential amplifiers, the output of this comparison beeing fed to two window

comparators driving the motors. The value of the (presently) 1M resistors

defines the rotator's dead range, so you can adjust it by modifying these

values. Lower resistance values cause the dead range to be  bigger, reducing

wear and tear, but worsening pointing accuracy. If you have stability problems

(oscillations), lower these resistances.

A switch is provided which allows disabling the FodTrack interface. When this

switch is ON, the interface will not allow you to move the rotator with the

pushbuttons on the controller. It will always hold the rotator to the position

stored in the interface. If you move the switch to the OFF position, you will

have full manual control over your rotator, while the interface still listens

to the PC and updates the D/A's internal registers.

As you can see, the loop is closed outside the PC, in hardware. This reduces

the processing load for FodTrack, and eliminates the possibility that a

computer crash may drive your rotator against it's end stop.

The program has no means to check if a rotator interface is connected, so you

can start it up without one.

The interface can be mounted in a small box. It is connected to the rotator

controller's 8-pin DIN connector, from which also the power is taken.

Please note that there are two standards in common use for the pinout of this

connector. The schematic diagram includes a picture of the proper pinout.

The program provides a calibration function which allows you to adjust your

rotator pots for correct ranging.

This sequence is valid for my interface circuits, and a Yaesu 5400 rotator:

- Assemble the interface, set everything up, load the program.

- Calibrate the rotator's full scale adjustments according to it's manual.

- Type c to get into the calibration routine.

- Enter 255 for both azimuth and elevation.

- Calibrate the rotator's output voltage adjustments to such positions that the

rotator stays just before it's end stops (or limit switches) in both axes.

- Enter 0 for both azimuth and elevation.

- Check that the rotator turns around completely in both axes, without running

against the limits. If it does run against a limit, something is wrong...

- Play around entering values of your choice, and make sure the rotator reacts

correctly (compare the meters to what FodTrack says).

Using the GS-232:

=================

Instead of the FodTrack rotator interface, you may use the Yaesu GS-232. This

allows to use FodTrack with ready made hardware for those who don't want to

build things. But of course the cost is much higher!

To use the GS-232, you simply set up the configuration file accordingly

(details are commented in that file), and connect the GS-232 to the selected

serial port. FodTrack uses only the Wxxx yyy command of the GS-232 and it does

not expect any answer, so you can use a very simple 2-line connection!

Radio Interface:

================

You can use the original RS232 interfaces, but these are costly. You can as

well use a simple level converter mounted inside a plug. Several firms offer

them at a much lower cost than the original ones, but it is still much cheaper

to build your own...

You really don't need a fancy MAX232 here. A simple CMOS chip, powered from 5

Volts, will do the job. Put a 100K resistor between the RS232 output and the

chip's input; that resistor together with the chip's clamping diodes will make

a very nice RS232 to TTL converter!

You can power the CMOS chip from a 78L05 regulator, fed through a few diodes

and a capacitor from the RS232 handshaking lines. FodTrack will "switch them

on" for you! You can put all these parts into an RS232 connector (even a 9-pin

shell is big enough, if you are careful...), to get a very nice and cheap

control interface.

Some radios use TRUE polarity, others need the RS232 signal inverted. Check the

docs of the radio, and use one or two gates of your 74HC04 accordingly.

Remember that for FodTrack you don't need data FROM the radio TO the PC, but if

you want this for some other program, just put the TTL signal into the PC's

RS232 input. I have never seen any port that does NOT work that way!

I'm providing the schematics of the adapters I use in my station. RS232Y.PCX

is the version for Yaesu, which I use on my FT736, while RS232K.PCX is for

Kenwood radios. I use that one for my TS450, and I hope it will run also on

the TS790, but I have not tested this.

I'm also providing a schematic for an interface which perhaps could get the

Guiness record for simplicity: just three components! This circuit does not

provide a "reply" connection for the radio, and it can be used only with radios

that have an input with pull-up, and use inverted signals. It works perfectly

well with FodTrack and the Yaesu FT736R, and will probably also work with

Icom radios (using another plug, of course...).

This circuit is in RS232SIM.PCX.

Ready made interfaces:

======================

For those who don't want to assemble their own interfaces, AMSAT-CE can provide

the circuits in FODTRK22.PCX and RS232SIM.PCX assembled and tested, ready for

"plug and play". Any funds collected go to our CESAR-1 project, a 9600 baud

Pacsat. If you are interested in this deal, write to Carlos Godoy, CE2HI, via

any of the following ways:

Mail:    AMSAT-CE

P.O.BOX 803

VINA DEL MAR

CHILE

e-mail:  ce2hi@entelchile.net  (This is Carlos' latest address. He has changed

the internet service provider several times

lately.)

Fax:     +56 32 88 4073

Cost, for the combination of the two interfaces, including shipping via

registered airmail, is about US$ 110. Unfortunately AMSAT-CE cannot charge

credit cards. The two interfaces can also be provided separately.

Automation:

===========

There are now two different ways for automatic tracking of multiple satellites.

The preferred mode is this:

Configure your skeduling program in such a way that it writes a FODTRACK.CMD

file into the FodTrack directory, containing the name of the satellite to

be tracked, exactly as it appears in the kep file. This CMD file should be

written two or three minutes before the start of the pass, to give FodTrack

enough time to do the crystal ball business and preset the antennas. The

easiest way to write these command files is simply to copy them from somewhere

elso into the FodTrack directory, using a COPY command inside the a batch file

you run at the start of a pass. For example, you can have a FODTRACK.CMD file

in your KO-23 directory, containing the text "KO-23" (without the quotes, of

course...). Into the BAT file you run at the start, you put the command

COPY C:\SAT\KO-23\FODTRACK.CMD C:\SAT\FODTRACK\FODTRACK.CMD

After the pass, the skeduling system should write a CMD file saying NONE, so

FodTrack stops.

Now you load FodTrack inside your multitasker. When your satellite skeduler

decides to run a pass, it calls the BAT file, which copies the proper

FODTRACK.CMD into the FODTRACK directory. When FodTrack reads the file within

the next second, it starts tracking the specified sat. After the pass, the

CMD file contents is changed to "NONE", and FodTrack goes to idle, waiting for

the next pass.

FodTrack checks the CMD file continuously, so it is mandatory to have a disk

caching program like SMARTDRIVE, or some hardware cache. Otherwise your hard

disk will complain. But anyway, nobody should be working without a cache these

days!

The alternative mode for automatization is simpler:

It consists just in configuring FodTrack for automatic exit after a pass (in

the CFG file), and calling it before each pass specifying the satellite on the

command line. For example, you use the command FODTRACK KO-23.

FodTrack will start up, wait for the specified sat, track it until the end of

the pass, and then quit. The command file is not read in this mode.

The disadvantage of this mode is that when FodTrack starts for a pass, it

doesn't know where the rotator is. This eliminates the possibility of selecting

flipping according to the status from the last pass, which is useful to reduce

rotator wear. In the preferred mode, with flipping and without parking, this

kind of rotator wear reduction works.

If you switch to manual mode after starting FodTrack with a sat on the

command line, it will then work in the normal way. If you then switch into

automatic mode, it starts reading the command file. But if you switch directly

from command line mode into automatic mode, the program will abort, because it

considers the pass finished.

If you use WISP, you can tell the event scheduler in GSC to do the command

file business via batch files, as described above. There is also a program

available, called FOD-INIT and written by CN8HB, which does the glueing job

between WISP and FodTrack. I'm not using WISP myself (don't like Windoze), so

I cannot give detailed instructions on this setup. If someone wants to provide

them, I will be glad to include them in the next version of this doc.

In case of trouble:

===================

If you have any trouble with FodTrack, tell me, so I can fix the bugs for the

next version. Also I will do my best to act upon any suggestion for

improvement, if reasonable and possible to implement.

To contact me, send a message on KO-23, KO-25, UO-22, AO-16 or LO-19. You can

also try through the terrestrial packet net, if my program shook your satellite

antennas to pieces. The address is XQ2FOD@XQ2FOD.SER.CHL.SA.

On Internet, you can reach me at mmornhin@eso.org. Be patient, sometimes

I'm away for several weeks.

Is there anyone around who likes to use snailmail? If so, use this address:

Manfred Mornhinweg

Radio Club Coquimbo

Casilla 381

Coquimbo

Chile

A last word:

============

I'm not a professional programmer, so you are allowed to smile about my program

and me...