RCICOMM.EXE is a small program Written By Mike Bilow N1BEE to test

for a Modems activity and to Dial a Modem that has been configured

as a port on a G8BPQ Packet Node.

If you enter RCICOMM at your DOS prompt without any commands you will

get this small help file:

RCICOMM: Write string to serial port (930326v1.32)

Copyright (c) 1993, Michael Bilow

Usage:

RCICOMM <port> <baud> [(<string> [<wait>]) | (DTR [0 | 1]) | (RTS [0 | 1])]

port 	Serial port address (0xNNN hex)

baud 	Baud rate to be set (decimal)

string 	String to be written to serial port

wait 	Seconds to wait after sending string (default: 0)

DTR 	Drop DTR for 10 seconds, then raise and leave up

DTR 0 	Drop DTR and leave down

DTR 1 	Raise DTR and leave up

RTS 	Drop RTS for 10 seconds, then raise and leave up

RTS 0 	Drop RTS and leave down

RTS 1 	Raise RTS and leave up

-- Baud rate is not changed if specified as zero

-- If neither DTR, RTS, nor <string> are given, only baud rate will be affected

-- The <string> must be enclosed in double quotes if it contains spaces or

special characters.

-- Code special characters of <string> \r for CR, \n for LF (like C programs)

-- If <string> is specified, DTR and RTS will be raised and left up

-- If wait not aborted, ERRORLEVEL is 0 if DCD not present, 1 if DCD present;

if wait aborted, ERRORLEVEL is 8 if DCD not present, 9 if DCD present

Example: 	RCICOMM 3F8 19200 "\rATDT4013310334\r" 30

I have used RCICOMM with Many diffrent Modems and Serial cards, It works

fine with shared IRQ's and weird addresses:  I am currently using a 4Com

serial card from STB with address 1A8 and 2A8 sharing IRQ 12 both with

Modems configured and RCICOMM is working fine.

If you have any problems or question send ME a note and I will try

to help you.

73 Steve SYSOP @ KA1RCI.RI.USA