TEKK RADIO NOTES

Collected by WÝTN

Rev. 7, 11-21-94

[WÝTN - What appears to be the definitive article on mods begins on p. 17.]

TEKK Data Division

800 521-TEKK

816 746-1098 (service)

816 746-1093 (fax)

TEKK Inc.

224 N.W. Platte Valley Dr

Kansas City MO 64150

[WÝTN - PacComm sells and supports them (813) 874-2980]

-----------------------------------------------------------------------

: 	TEKK Incorporated

: 	228 NW. Parkway

: 	Kansas City, MO  64150

[Old address? -W0TN]

: 	(816) 746-1098

: 	Inquire about the TEKK Micro.  It's $90.00  You can request

: crystals for the radio.  We in Buffalo Decided on 441.050.  Ask for a

: service manual when you order it.  (They don't send any paperwork, not

: even pinouts!  Although a BNC connector is rather obvious, you may

: stumble when trying to trace back the DB9 connector.)  If you decided

: on the TEKK Mini, before you try and do anything with it, remove

: Capacitor C18.  It's SMT, so be sure to have a fine soldering iron and

: no coffee that morning.

Also, TEKK will swap out MINI's for those who got them by mistake.  The Mini

rolls off the receive audio frequency response - that's why the capacitor

has to be removed.  It also modifies the transmit passband and implements

deviation limiting.  Unfortunately the deviation limiting is set for 5 Khz,

not 3 Khz that is mandatory for normal 9600 bps operations.  I've also had

some reports from local hams that TEKK has been modifying their pricing.

Specifying you saw the ad in QST will probably get you the best price.  They

are now charging for crystals, which were included in the purchase price 3

months ago.

: 	CAUTION!!! Do not run the radio at more the 12.0V until you're

: sure I'm not kidding.  CAUTION!!!

: 	Next, get an MFJ TNC, (1270B Personally) about $140.00, and

: the MFJ 9600 Modem.  Another $100.00

Or an PK-96 or a DRSI DPK-96 and no modem installation - also have heard

that Kantronics has new one out (KPC-1296?) that is ready to go out of the

box.  If you don't already have a TNC that can accept an add-in modem, it's

no longer cost effective to buy a TNC and a modem as separate items.

Several locals have gotten PK-96's and had no problems getting them up and

running with TEKK's and some of the other known good 9k6 radios.

PK-96's are retailing at $200 +- $10-15  (list is higher, of course).  I

have not seen or heard pricing on Kantronics late offering.

: 	Install the modem. (READ THE MANUAL FIRST!  Believe me,

: they're "Instructions", not "distructions"!)

------------------------------

> John, WB3ESS

> aljkbe@attme.att.com

Try Piezo Technology  (Technologies??) in the Ft. Lauderdale, FL area.

Sorry, I don't have their phone number, but LD information for Area Code

305 should be able to find it for you.  As of a few years ago they had

filters for both 10.7 and 21.4 in several different bandwidths and options for sharp skirts or minimal passband ripple.  At that time they were the best US supplier I could find for such goodies.

--

Karl Beckman, P.E.                  <     If our English language is so  >

Motorola LMPS.RNSG.Analog Data      <  precise, why do you drive on the  >

(Square waves & round corners)      <  parkway and park on the driveway? >

Opinions expressed here do not belong to or represent Motorola Inc.

Amateur radio WA8NVW                        NavyMARS NNN0VBH @ NOGBN.NOASI

----------------------------------------------------------------------

Here are some initial notes I made while looking at a TEKK Mini for the

first time.  At the time, I had no schematic or manual.  ab6ym has since

FAXed me one and I know a bit more than I did.  If I get time, I hope to

measure the receiver to see what the cause of it's recovery problem is.

First glance might make one wonder about the wisdom of shutting of a

crystal oscillator and expecting it to come up rapidly.

The Mini and Micro appear to be different primarily in the receiver

chip they use (There are some physical differences but I'm just

comparing schematics) I wouldn't expect a great deal of strong signal or

sensitivity and distortion differences between the two based on what the

schematics show.

Executive summary is that except for receiver recovery time, they seem

to perform as advertised.  It appears to me that one needs to be quite

careful to be sure that they are netted (and don't drift) and that

modulation levels are kept low to avoid the edges of the the filters,

particularly the 21.4 MHz 1st IF filter. Spurious and strong signal performance

(I didn't properly measure desense due to noise floor of the particular sig

gen I was using) seem to put them in the same class as much other amateur

equipment.  I did not measure TOI.

My first notes follow.

Glenn n6gn

---------------------------Initial TEKK Mini Notes-------------------------------

Receiver

---------

DCD fully asserts in about 30 milliseconds after RF  comes true

RSSI saturates around -65 dBm, floor is below -110 dBm

overall 3 dB bw about 15 KHz, limited by 21.4 filter

21.4 filter is flat, ~1 dB p-p ripple

but with 50 us group delay unflatness

15 KHz between horns, fairly uniform "bowl"

455 KHz filter is 25+ KHz wide

4 dB p-p ripple (maybe my connection messing it up)

also around 50 us GD ripple but not a bowl, 20 KHz between horns

1st  LO=45.781X9+21.4=433.43 MHz

2nd  LO=20.945 -21.4= 455 KHz

Image of 21.4 IF at 390.63 is down 70 dB

Image of 455 IF at 432.52 is down 70 dB

Desense floor was lower than the (poor) sig gen I was using (broadband noise

and I didn't have a 433.43 notch handy)

Transmitter

---------

+33 dBm (2W) at 9.6V @ 600 mA

Full RF power in around 1 millisecond after asserting PTT

Second harmonic ~-60 dBC

several sets of spurs +- .5-25 MHz down around -70 dBC

no obvious broadband noise or humps.

130 mv P-P of TxAudio "fills" +- 5KHz passband with 4 dB peaks at

edges (TAPR scrambling as waveform).  This gives +-7.5 KHz down about

-20 dBC and receiver is recovering 180 mv P-P which compares to 320 mv

p-p on noise.  This seems to be a reasonable place to run it once

everything is netted considering the above filter responses.  Increasing

the TxA causes noticeable non-linearity in recovered audio as well as some

distortion in the transmit spectrum.  I'm not sure which is contributing

the most, tx or rx, but running higher deviation seems a bad idea.

As expected, running TxDelay much lower than 40 ms starts to cause

problems.  AX25 loopback connection on bench maintained at 30 but not 20

ms.  Didn't measure but I'd anticipate that weak signal performance is

compromised even at 30.  As noted before, the TEKK DCD performance could

use some speedup.  Don't know whether it is a problem with recovering

from saturation, power supply switching or and without a schematic I'm

not anxious to figure it out at the moment.  Still, this is an area for

considerable improvement.

40 ms at 9600 bps is 48 bytes overhead due to keyup/sense delay.  40

ms at 9600 bps is 48 bytes overhead due to keyup/sense delay.

-----------------------------------------------------------------------

Attached you will find modifications for both model TEKK radio's that improve

the receive and transmit performance at 9600bps. It's reported that both TEKK

and PacComm have been making some/all of these modifications. I have no idea

when they implemented these changes. Receiver IF tuning is important with

both model receiver. If anyone can suggest a source for 1st IF crystal filters

for these radio's please post it. Piezo Technologies makes filters however,

they quoted me a price of $60.

John, WB3ESS aljkbe@attme.att.com

TEKK Data Radio Mods for 9600 bps

by John Bednar, WB3ESS

Original Issue:12/93

Last update: 5/16/94

TEKK KS-900L Receiver Mods

At the 9-pin interface connector add a 2.7k ohm resistor in

series with the wire going to pin 5. Next, solder a 0.001uf

capacitor between pin 2 and pin 5 of the same connector.

TEKK KS-900L Transmitter Mods

Solder a 3.3uf tantalum capacitor (observe the polarity) in

parallel with capacitor C31. The transmit requirements are not

changed so re-adjusting the transmit level will not be required.

TEKK KS-960L Receiver Mods

Remove chip capacitor C18 and replace it with a 0.001uf leaded

capacitor or chip capacitor.

TEKK KS-960L Transmitter Mods

Bypass the transmit limiter, modify the modulator, and inject the

audio into TP9 with the following steps:

1) Remove the wire attached to pin 4 of the 9-pin connector and

solder it to ground.

2) Solder the (+) lead of a 10uf tantalum capacitor with 0.2"

leads to the 9-pin connector - pin 4.

3) Prepare a 1.8" piece of wire with 0.2" ends and solder one end

of the wire to the (-) lead of the capacitor added in step 2.

Tuck the other end of the wire around the circuit board in the

corner. Don't use the corner where the BNC is mounted.

4) Locate the junction of C43, C92, and R41. Form the wire, trim

the end to 0.1" and solder it carefully to this junction.

5) Locate the junction of R43 and C47. Carefully remove the

solder from that solder pad. With a sharp knife cut the trace

diagonally between R43 and C47. Check your cut with an ohm

meter to insure that the trace is open. If you break R43, C47,

C48 or C49 don't worry because these parts are no longer used.

6) Locate potentiometer VR1 and turn it fully CW.

7) Re-adjust the transmit frequency and the transmit deviation.

A complete article with frequency response graphs will appear in

the January 94 Northeast Digital Association Quarterly. As of 10/94

this issue has not been printed.

------------------------------------------------------------------

>

> >

> > I am looking for a source of 1st IF crystal filters

> > for TEKK data radios. I'd like to change the receive

> > filters and experiment at higher data rates. I have a

> > source for the 455 khz filter but I'm still looking

> > for the 1st IF filter.

> >

> > Piezo Technologies makes filters however, they quoted me

> > a price of $60 !!

> >

> > John, WB3ESS

>

>

>  I suspect that higher price is going to be the rule rather than the

> exception for quartz filters of this type unless someone can find a

> high volume application that already uses something 30-50KHz wide.

>   If it will help, John, I think I can come up with a few 30 KHz wide

> filters centered on 20.0 MHz.  Along with your wider 2nd IF filters

> this would let you try things out at 19.2 kbaud.  However, I don't have

> a large quantity source for these, they're just something I think I have

> stuffed away in a drawer.

>

> 73

> Glenn Elmore n6gn

>

> amateur IP:	glenn@SantaRosa.ampr.org

> Internet:	glenne@sr.hp.com

>

One potential mass-market application is television audio-only receivers.

TV uses a peak deviation of +- 25 kHz, so you might end up with a 70 Khz

filter.

73, Bob

----------------------------------------------------------------------------

|  Bob Donnell, KD7NM     bob@ethanac.kd7nm.ampr.org   rdonnell@eskimo.com |

|  Western Washington Amateur IP Address Coordinator   (206) 775-3651      |

----------------------------------------------------------------------------

----------------------------------------------------------------------------------------

From: 76576.2003@compuserve.com (Gwyn Reedy W1BEL)

Date: 01 Nov 94 07:22:19 EST

To: <tmoulton@ram.net>

Subject: Re: Forward: [NETSIG:142] Re: TEKK 1st IF filters

Reply-To: 76576.2003@compuserve.com

PacComm stocks and sells filters for the TEKK radios. We have filters for

better 9.6 operation and for 19.2. Send for a catalog for details. Prices

are well below $60, but if you change all the filters you're still in the $40

class.

PacComm    To order a catalog call 800-486-7388 or send e-mail

---------------------------------------------------------------------------------------

On Nov 1,  5:46pm, Bruce Perens wrote:

> Subject: [NETSIG:195]  Re: Users and 9600 baud

> Karl said:

> > So my point is this: Can you say the TEKK KS-900 radio is 9600

> > baud ready? I say no. You must include a lot of information.

>

> I think the problem may only exist with the KS-960 (the chrome one), and

> not the KS-900. Has anyone ever had to swap filters in a KS-900? If nobody

> has, we've an easy answer.

>

> I bought a KS-900 a month ago, and a KS-960 last year, and both

> seem to work fine on 9600. Both were direct from TEKK.

I just plugged my KS-900 into the electricity, the Data Engine,

and the antenna, and it has worked just fine ever since.

...  Hank

--

Hank Oredson @ Mentor Graphics             Library Operations

Internet     : hank_oredson@mentorg.com    "Parts 'R Us!"

Amateur Radio: W0RLI@W0RLI.OR.USA.NOAM

------------------------------------------------------------------------------------

> Karl said:

> > So my point is this: Can you say the TEKK KS-900 radio is 9600

> > baud ready? I say no. You must include a lot of information.

>

> I think the problem may only exist with the KS-960 (the chrome one), and

> not the KS-900. Has anyone ever had to swap filters in a KS-900? If nobody

> has, we've an easy answer.

>

> I bought a KS-900 a month ago, and a KS-960 last year, and both

> seem to work fine on 9600. Both were direct from TEKK.

> 	Bruce Perens AB6YM

Yes, the older black cased TEKK works right out of the box at 9600

however, you can improve the performance by making the mods posted.

Bruce, you might check your chrome cased TEKK and let us know if the

TX limiter has been by-passed by TEKK. The original units worked at

9600 bps but performance was poor. I purchased mine in early 12/93

(limiter not by-passed).

With the posted modifications to the chrome cased TEKK the

performance greatly improved.

If performance is still poor after the modifications, check

the IF tuning. We had one TEKK that was out of tune and it was

abnormal until correctly tuned. We caught that one by measuring the

RX frequency response vs. frequency.

John, WB3ESS

-------------------------------------------------------------------------

On Wed, 2 Nov 1994, Bruce Perens wrote:

> However, we have an answer to one of Karl's questions. We can tell the

> new high-speed operators to buy a KS-900 rather than a KS-960. Thanks!

I bought the KS-900 and it works fine...  I did the mods that John told

about and it helped a little bit on weak signal receive...

73, de Bill, N3JIX

******************************************************************

* Bill Barnes, N3JIX                    * Bloomsburg University  *

* Internet:  wlbarn@planetx.bloomu.edu  * Bloomsburg, Pa         *

* Packet  :  n3jix@n3jix.#epa.pa.usa.noam                        *

******************************************************************

--------------------------------------------------------------------------------------

Good suggestion Bruce.

Hows's this?  The KS960 will be re-done later.

John, WB3ESS

TEKK Data Radio Mods for 9600 bps

by John Bednar, WB3ESS

These modifications are not for the TEKK model KS-960 (the

radio in the chrome case).

TEKK KS-900L Receiver Modification

The KS900 recovered audio has a high frequency (2.2MHz) signal

superimposed on the recovered audio. This high frequency signal

causes jitter in the sliced output. To eliminate the unwanted

signal, install a lowpass RC filter in the audio signal line

with a corner frequency well above the frequencies of interest.

The modification: At the 9-pin interface connector add a 2.7k

ohm resistor in series with the wire going to connector pin 5.

Next, solder a 0.001uf capacitor between pin 2 and pin 5 of

the same connector.

TEKK KS-900L Transmitter Modification

The stock KS900 transmit modulation response is not flat from

"dc to 7.2 Khz". To correct this deficiency, the input coupling

capacitor must be increased in value. I have found it easier to

solder in the additional capacitor without removing the board

from the case.

The modification: Solder a 3.3uf tantalum capacitor (observe

the polarity) in parallel with capacitor C31. The transmit

requirements are not changed so re-adjusting the transmit

level will not be required.

-------------------------------------------------------------------------

TEKK Data Radio Mods for 9600 bps

by John Bednar, WB3ESS

These modifications are for the TEKK model KS-960 (the

radio in the chrome case).

These modifications were developed in December 1993

and sent to TEKK in early January 1994. I have received

correspondence that indicates both TEKK and PacComm are

currently modifying radios with this procedure. Check your

radios to verify that all steps are completed.

TEKK KS-960L Receiver Modification

The un-modified KS960 recovered audio is attenuated by

a single pole RC filter on the receiver output. The roll-

off begins around 1200hz in an unmodified radio. Please

note that a simple low-pass filter is required to eliminate

a high frequency (2.2MHz) conversion product. The solution

is to move the corner frequency of this RC filter well

above the frequencies of interest.

The modification: Remove chip capacitor C18 and replace

it with a 0.001uf leaded capacitor or chip capacitor.

TEKK KS-960L Transmitter Modification

The un-modified KS960 transmitter suffers from several

problems. First, this radio contains an input amplifier,

a limiter, and a reconstruction filter in the input of the

transmit section. Secondly, the modulator frequency response

is not not flat from "dc to 7.2 Khz".

The solution is to bypass and ground the input to

the existing limiter circuit and to modify the modulator

to achieve a flat frequency response.

The modification:

1) Remove the wire attached to pin 4 of the 9-pin connector and

solder it to ground.

2) Solder the (+) lead of a 10uf tantalum capacitor with 0.2"

leads to the 9-pin connector - pin 4.

3) Prepare a 1.8" piece of wire with 0.2" ends and solder one end

of the wire to the (-) lead of the capacitor added in step 2.

Tuck the other end of the wire around the circuit board in the

corner. Don't use the corner where the BNC is mounted.

4) Locate the junction of C43, C92, and R41. Form the wire, trim

the end to 0.1" and solder it carefully to this junction.

5) Locate the junction of R43 and C47. Carefully remove the

solder from that solder pad. With a sharp knife cut the trace

diagonally between R43 and C47. Check your cut with an ohm

meter to insure that the trace is open. If you break R43, C47,

C48 or C49 don't worry because these parts are no longer used.

6) Locate potentiometer VR1 and turn it fully CW.

7) Re-adjust the transmit frequency and the transmit deviation.

------------------------------------------------------------------------

>Can the KS-900 be used for 1200 baud and 9600 baud at the same time easily?

>

>I'm looking interested in getting a 9600 bps ready radio with a KPC9612,

>right now the backbone is only 1200 baud, but I want to be ready for 9600.

>(of course down the road I might swap the KPC9612 for a 9600 bps only TNC,

>but for now I just want to play around with the possibility.....)

The stock KS-960 (chrome case) works well at 1200 baud and contains a

microphone amp.

When you get ready to upgrade perform the mods for 9600...

Mike N4CNW,  mamurphree@space.honeywell.com

-------------------------------------------------------------------------

Brian Battles, WS1O    writes:

> I'm confused; the 900 is the one in the black case, right? And the 960 is in

> the chrome case, yes?

>

> I have a 900 that I got with my Gracilis PackeTwin, and I'd like to see if I

Brian,

I'm glad you posted this.  I'm confused too.  In fact, I think there

is a whole lot of confusion WRT the TEKK radios, model numbers and what

modifications do or do not need to be done.

I recently examined a pair of radios recently (Sept '94) purchased from

TEKK which were in a chrome case.  I also obtained two sets of

documentation from Bruce Perens.  One set was for the "T-Net Micro",

models KS900 and KS900L.  The other was for the "T-net Mini" and

*apparently* model number KS-900N.  The radios I looked at matched the

"T-net Mini" description.  I don't have a copy of the shipping invoice

here to tell what TEKK called them.

I've previously posted my measurements of these radios' performance

and on the differences indicated between them by the "T-net Micro"

documentation.  The primary difference appears to be in the receiver IC

and the lack of RSSI and associated DCD circuits in the "Micro".  With

regard to 9600 bps operation, both appeared to be limited by the

characteristics of the same 1st IF (21.4 MHz) crystal filter which is

about 15 KHz wide in amplitude and perhaps a tad less between group

delay "horns".

The radio I examined already appeared to have the tx audio limiter

circuits entirely bypassed and I found that none of the previously

reported modifications appeared necessary.  The even order products of

the last IF were not "riding" on the output audio.  The only

modification that might apply was in the low frequency response of the

Tx audio which I didn't measure (though BER tests with a TAPR modem

appeared satisfactory).

So before I would jump in making assumptions about suitability of one

radio over the other or about what modifications the radios do or don't

need, I would ask "someone who knows", what is the version history of

the TEKK radios, what changes have been made in their production and

when did they occur. Maybe that way, we can all be on the same page.

73

Glenn Elmore n6gn

amateur IP:	glenn@SantaRosa.ampr.org

Internet:	glenne@sr.hp.com

-----------------------------------------------------------------------

We just modified two KS-960 that were purchased 11/93. The mods

made a big difference. Did not find that the 10uF cap in the rx data

line made any difference, what is its purpose? Using the radios with

DPK-9600 tncs. A third KS-960 purchased 4 weeks ago seems to work

quite well out of the box but it seems its txdelay needs to be abt 5

where the older TEKKs are reliable with the txd @2-3.

Sperry/n4sd

InterNet:  sperry@pinn.net

Amprnet:   n4sd.ampr.org [44.62.64.40]

PBBS:      N4SD@WD4MIZ.#TIDE.VA.USA.NOAM

----------------------------------------------------------------------

The KS-900L is for 430-450, the KS-900H is for 450-470. These are

in black boxes, do not have space for an internal modem, and are

also called "T-Net Micro".

The T-Net Mini service manual uses different nomenclature than what

actually appears on the radio. The FCC ID in the manual is "GOXKS-900",

the same as the one for the other radio. The schematics say "KS-900N".

However, the sticker on the bottom of the radio identifies it as a

KS-960, and the FCC ID on the sticker is "GOX KS-960". This is the

silver box with space for TEKK's 4800 modem (but not enough space for

any other modem I know) and windows for 4 LEDs to show through the

front label.

My KS-960 has serial number 6448, while my KS-900 has serial number

9911. Both radios have an orange dot applied to their labels, and came

in paper boxes that also had an orange dot applied to them. I don't

have any idea of their revision history, and what modifications have

already been done in the factory.

Bruce Perens AB6YM

-----------------------------------------------------------------------

>  I recently examined a pair of radios recently (Sept '94) purchased from

>Tekk which were in a chrome case.  I also obtained two sets of

>documentation from Bruce Perens.  One set was for the "T-Net Micro",

>models KS900 and KS900L.  The other was for the "T-net Mini" and

>*apparently* model number KS-900N.  The radios I looked at matched the

>"T-net Mini" description.  I don't have a copy of the shipping invoice

>here to tell what TEKK called them.

The KS900 and KS900L designations are now called a TNET MINI

and comes in a black case.  The "L" indicated the low UHF range option.

The KS960 radio is now called the TNET MINI and comes in a chrome case.

The MINI has a fancier IF chip with RSSI output and a larger case to

accommodate a modem.

Other observations...

The MINI has cheaper RF final in it!  The final in the Micro is rated at 5W

vs. a 3W final in the MINI.  Because both the RX and TX low-level stuff in these

radios have regulators and the finals are 12V models, they will work just

fine from a  12-14V source.  I routinely run the Micro at 12+ V to boost the

output power delivered to an external amplifier. Since there is no output protection,

if you anticipate antenna problems, it might be safer to stick with the 9.6V supply.

Your mileage may vary,

Mike N4CNW, mamurphree@space.honeywell.com

--------------------------------------------------------------------

Mike, N4CNW writes:

>

> The KS900 and KS900L designations are now called a TNET MINI

^^^^ don't you

mean "MICRO" here?

> and comes in a black case.  The "L" indicated the low UHF range option.

documentation says "L" is 430-450 MHz.

> Other observations...

>

> The MINI has cheaper RF final in it!  The final in the Micro is rated at 5W

> vs. a 3W final in the MINI.  Because both the RX and TX low-level stuff in these

> radios have regulators and the finals are 12V models, they will work just

> fine from a  12-14V source.  I routinely run the Micro at 12+ V to boost the

> output power delivered to an external amplifier. Since there is no output

> protection, if you anticipate antenna problems, it might be safer to stick with the

> 9.6V supply.

I'm not clear on the differences in the output sections. The MICRO

claims to use an MRF652 which has planar leads and (potentially)

7 deg/watt thermal characteristics. The docs for the MINI claim it

uses an MRF830 which is not shown in Motorola's recent Master Catalog nor

in a couple of revs of the  RF Devices catalog.

As I remember, the MINI uses a device in a TO-5 package and it

*probably* doesn't have as good thermal characteristics, but it may

depend on how they've mounted it compared to the MRF652 in the MICRO.

I would caution folks about running higher Vcc on these radios.  On

the MINI I examined, I found that there was a tendency toward

instability, particularly at higher Vcc.  Large spurious sidebands could

be generated with only slight tuning changes very close to a maximum

output condition.  This was into a 50 ohm resistive load, it could very

possibly be even worse with mismatched loads.

I think some analysis of spectral purity and thermal performance is in

order before remoting one of these at a high level site with >12 VDC

supply.  It probably would be a good idea to verify things no matter

what supply V one uses.

Glenn Elmore n6gn

amateur IP:	glenn@SantaRosa.ampr.org

Internet:	glenne@sr.hp.com

--------------------------------------------------------------------

>> The KS900 and KS900L designations are now called a TNET MINI

>

>                                                         ^^^^ don't you

>                                                         mean "MICRO" here?

GEEZ!   Now you've got me doing it.... ;^)

>  I'm not clear on the differences in the output sections. The MICRO

>claims to use an MRF652 which has planar leads and (potentially)

>7 deg/watt thermal characteristics. The docs for the MINI claim it

>uses an MRF830 which is not shown in Motorola's recent Master Catalog nor

>in a couple of revs of the  RF Devices catalog.

My databook here at work doesn't have the MRF830, but I keep my GOOD

databooks at home...  And I do have one somewhere that has it. Thermal

resistance, Junction to Case is indeed 7 measly degrees per watt. Max Vceo

is 16 VDC, Max device dissipation is 25 watts, max junction temp is 200 deg

C.

>  I would caution folks about running higher Vcc on these radios.  On

>the MINI I examined, I found that there was a tendency toward

>instability, particularly at higher Vcc.  Large spurious sidebands could

>be generated with only slight tuning changes very close to a maximum

>output condition.  This was into a 50 ohm resistive load, it could very

>possibly be even worse with mismatched loads.

No excuse for not verifying proper operation of your equipment, this is

true.

Obviously one shouldn't expect too much out of $100 radios, but my general

impression of the *MINI* was that it was of poorer quality than the *MICRO*,

even though it costs MORE. Layout, parts quality, and out-of-the-box

performance were less. I have not seen any stability problems with the

radios  operating at 12V, though the same could not be said for a

particular manufacturer's TNC.

>  I think some analysis of spectral purity and thermal performance is in

>order before remoting one of these at a high level site with >12 VDC

>supply.  It probably would be a good idea to verify things no matter

>what supply V one uses.

One must enjoy travel if they are going to put one of these things at a

remote site.  Some of the surplus commercial gear would be a much better

choice. Tekks are not bad as end user radios and for small links but I

wouldn't use them in critical applications.

Mike N4CNW

-----------------------------------------------------------------------

>We just modified two KS-960 that were purchased 11/93. The mods

>made a big difference. Did not find that the 10uF cap in the rx data

>line made any difference, what is its purpose? Using the radios with

>DPK-9600 tncs. A third KS-960 purchased 4 weeks ago seems to work

>quite well out of the box but it seems its txdelay needs to be abt 5

>where the older TEKKs are reliable with the txd @2-3.

>

>Sperry/n4sd

>  InterNet:  sperry@pinn.net

>  Amprnet:   n4sd.ampr.org [44.62.64.40]

>  PBBS:      N4SD@WD4MIZ.#TIDE.VA.USA.NOAM

>

>

Sperry,

We've found over time that the 10uf cap is important to

achieve good low frequency response for a low BER at 9600.  Some

(the better) radios will have this on their data output lines as

they come from the factory.

A longer TXdelay can actually indicate slow receiver

recovery time.  If the link works with UI packets, but needs a longer

TXD when connected, that usually corresponds with slow-ish rx

recovery.

Regards,

Andy KC2FF

DRSI

----------------------------------------------------------------------

On Mon, 7 Nov 1994, D. Crompton wrote:

> I just received two TEKK radios and they came in a pink case!

> What mods should I make on them?

>

> Doug

>

>

Quickly paint them flat black and call them TEKK KS-900. Have you

looked inside yet Doug? And what did they cost now? I bought 4 KS-900

units when they were $89.00 a piece if you took the 441.05 MHz crystals.

Since we have no simplex bandplan in NM, I am using that frequency on

packet at 9600 baud...

Try this: Fire them up from a 9 volt source. Using your DJ580

Handy Talky with a scope on the receiver output make a signal on

frequency. Set that as the center of the passband. Then make a signal 5

KHz low and note the deflection on the scope. Now make a signal 5 KHz

high and note that deflection. If the center of the passband is not the

center between deflections, then your radio isn't receiving on the right

frequency, or your radio is off frequency (not possible if you have a

DJ580). Tune the receiver so you have equal deflection and when you hit

the zero on your touchtone keypad you should see a sine wave that is for a

3 KHz deviation.

If all that looks good then they will receive 9600 baud fine. If

not look at all that stuff about modifying the ones not set for data...hi

The transmitter should make 2+ watts on the right frequency.

-karl

-----------------------------------------------------------------------

On Tue, 8 Nov 1994, Bruce Perens wrote:

> Karl Larsen wrote:

> > he wants not my help.

>

> I don't blame him. Your touch-tone method of setting deviation wasn't

> meant to be a joke, was it?

>

> 	Thanks

>

> 	Bruce Perens AB6YM

>

Bruce the touch-tone method of setting deviation is not a joke. I

first took my ht to a service monitor and had it's touch-tone deviation

measured (it is almost exactly 3 KHz).  I then calibrate my scope image

in terms of p-p deflection with the ht making 3 KHz sinewaves. Then I

look at the signals I am connected to and note their deflection. Using

this method I discovered that the mountain-top node has a 1 KHz deviation

now and I will get up there on Monday next to fix it.

I should say the scope is connected to the Tekk receiver audio output

line. Any DC scope will work. This is as accurate as it gets Bruce. The

only thing better is a $12,000.00 service monitor...

-karl

-------------------------------------------------------------------------------------

From: Murphree Michael A <mamurphree@space.honeywell.com>

> I pulled out my schematic for my TNET Mini and the identification

> number there was KS-900N.

The model number in the T-Net Mini service manual is incorrect. I did

mention this in my first posting on TEKK Nomenclature. If you already

own a radio, it's up to you to determine what modifications it needs.

If you are about to purchase a radio, I can make it easier for you:

I haven't been able to find any reason for a radio amateur to purchase

a KS-960. The KS-900 works acceptably well for 9600 right out of the

box. The KS-900 may benefit from some small modifications, but these

modifications are trivial compared to the major surgery proposed for

the KS-960. The KS-960 runs hotter, has a lower-power final output

transistor, is larger in size, and costs $10 more than the KS-900.

The only known advantage of the KS-960 is that it has a S-meter output,

but this output responds too slowly to make any practical use of it on

packet. Buy a KS-900 instead.

Again, I am ignorant of the design-change history of these radios, and

can not tell you what features are disabled in current models, and what

modifications are necessary. I can, however, tell you that a new KS-900

works reasonably well as soon as you plug it in.

Thanks

Bruce Perens AB6YM

-----------------------------------------------------------------------

My mailer said Tim Baggett said this:

>

> > of my head but will not. He has locked me out of all his e-mail accounts

> > so he wants not my help.                             ^^^

> >                                                       the magic word

> Now, back to my question: Anyone know the scoop about which radios are

> which? I'm looking for answers - not harassing goofs.

After all these discussions I popped off the cover of a TEKK radio here

that required the mod. It's the usual chrome case KS-960.  When I first

tried 3 of these radios I got about 8-11 % thruput.  After the mods were

done they were error free at 9600.  I brought the radios to a friends

house to mod them (he had the tools for SMT work) and it took about 10

minutes for the first radio and 5 min for each other radio.  Total time

for everything was just under 30 minutes to do 3 radios.  This assumes

though that you have the right equipment, like a low wattage soldering

iron with a very small tip or one for SMT work.

The 3 TEKK's here are all KS-960 models with serial numbers in the 8000

range.  The pc board itself has the designation:

KS-960N

KS-900N

Yes, it does have both of those numbers laid out that way on the board.

With 2 TEKK's talking to each other they run just great with about 5

for a txdelay, but I've run it down to 3 and I notice it starts to

occasionally miss a packet.  5 is always error free though.  Amazingly

my Kenwood TM-731A dual-band performs right along with this thing

except the txd needs to be a bit higher (can run about 8-10) and it

decodes the TEKK's just fine when they run fast txdelays. It sure seems

to outperform Alinco radios running 9600 (haven't found an Alinco yet

that can decode me at 8 for a txd). But my Kenwood is just for home

packet use to goof around with.

Personally if I bought another one I'd just order a KS-900 TNET-Micro

after hearing from others on here.  These radios work just fine though.

--

Ron N8FOW

AMPRnet  : n8fow@n8fow.ampr.org

Internet : ron@chaos.eng.wayne.edu

aa011@detroit.freenet.org

------------------------------------------------------------------------

Tim,

Here is the scoop on the TEKK mods. I am the author of the mods

and original postee.

John, WB3ESS

PS:

Does anyone want to work on a TEKK (19.2 k or 38.2 k) mod? I haven't

done much work but having a buddy or two might keep the

project moving. My Internet address is aljkbe@attme.att.com.

===================================================================

The older style radio has a black case that only has a top

cover. If you make these mods to the radio you will find an

improvement in 9600 bps operation. Yes, the radio works right

out of the box, but you *will* see an improvement by adding two

caps and one resistor. It's a very simple mod I can't understand

why some readers are grumping. The modification was developed because

this radio was not developed for 9600 bps service and I wasn't

happy with the stock performance.

TEKK KS-900L Receiver Modification (black case with only a top cover)

The KS900 recovered audio has a high frequency (2.2MHz) signal

superimposed on the recovered audio. This high frequency signal

causes jitter in the sliced output. To eliminate the unwanted

signal, install a lowpass RC filter in the audio signal line

with a corner frequency well above the frequencies of interest.

The modification: At the 9-pin interface connector add a 2.7k

ohm resistor in series with the wire going to connector pin 5.

Next, solder a 0.001uf capacitor between pin 2 and pin 5 of

the same connector.

TEKK KS-900L Transmitter Modification (black case with only a top cover)

The stock KS900 transmit modulation response is not flat from

"dc to 7.2 Khz". To correct this deficiency, the input coupling

capacitor must be increased in value. I have found it easier to

solder in the additional capacitor without removing the board

from the case.

The modification: Solder a 3.3uf tantalum capacitor (observe

the polarity) in parallel with capacitor C31. The transmit

requirements are not changed so re-adjusting the transmit

level will not be required.

===================================================================

===================================================================

The newer style radio has a chrome case. It uses chip components

and the case has a top and bottom cover. The model numbering of

this radio is very confusing. When these radio's were originally

shipped, the thruput at 9600 bps was poor but they also worked out

of the box (much worse than the older style radio).

If you make these mods to the radio you will find an improvement

in 9600 bps operation. This modification is a little more

detailed but don't let the quantity of text prevent you from performing

the change.

Now here is the interesting part. I released the modification to TEKK

around early January 1994. Naturally, it has found it's way to the

other suppliers of TEKK radio's. Based on correspondence I believe that

some suppliers are *not* making all the modifications. If you want

the maximum performance at 9600 bps and near identical frequency response

to the older style radio, I contend that all modifications must

be completed. It is possible to complete some of the modifications

and see a dramatic improvement in performance *but* with all the

changes, performance will improve.

I would expect by now that TEKK is shipping radio's that work *well*

out of the box. There is no need to stay away from this radio on a

frequency response standpoint. Someone mentioned RF transistor concerns

a few days ago. So please factor this into your purchase.

TEKK KS-960L Receiver Modification

The un-modified KS960 recovered audio is attenuated by

a single pole RC filter on the receiver output. The roll-

off begins around 1200hz in an unmodified radio. Please

note that a simple low-pass filter is required to eliminate

a high frequency (2.2MHz) conversion product. The solution

is to move the corner frequency of this RC filter well

above the frequencies of interest.

The modification: Remove chip capacitor C18 and replace

it with a 0.001uf leaded capacitor or chip capacitor.

TEKK KS-960L Transmitter Modification

The un-modified KS960 transmitter suffers from several

problems. First, this radio contains an input amplifier,

a limiter, and a reconstruction filter in the input of the

transmit section. Secondly, the modulator frequency response

is not not flat from "dc to 7.2 Khz".

The solution is to bypass and ground the input to

the existing limiter circuit and to modify the modulator

to achieve a flat frequency response.

The modification:

1) Remove the wire attached to pin 4 of the 9-pin connector and

solder it to ground.

2) Solder the (+) lead of a 10uf tantalum capacitor with 0.2"

leads to the 9-pin connector - pin 4.

3) Prepare a 1.8" piece of wire with 0.2" ends and solder one end

of the wire to the (-) lead of the capacitor added in step 2.

Tuck the other end of the wire around the circuit board in the

corner. Don't use the corner where the BNC is mounted.

4) Locate the junction of C43, C92, and R41. Form the wire, trim

the end to 0.1" and solder it carefully to this junction.

5) Locate the junction of R43 and C47. Carefully remove the

solder from that solder pad. With a sharp knife cut the trace

diagonally between R43 and C47. Check your cut with an ohm

meter to insure that the trace is open. If you break R43, C47,

C48 or C49 don't worry because these parts are no longer used.

6) Locate potentiometer VR1 and turn it fully CW.

7) Re-adjust the transmit frequency and the transmit deviation.

----------------------------------------------------------------------

>John, WB3ESS

>

>PS:

>Does anyone want to work on a Tekk (19.2 k or 38.2 k) mod? I haven't

>done much work but having a buddy or two might keep the

>project moving. My internet address is aljkbe@attme.att.com.

>

I think PacComm has done atleast 19.2kb. that would be a fair starting point.

tom, w2vy

-----------------------------------------------------------------------

From: Mark Stappenbeck <froggy@southwind.net>

> Could anyone tell me whether the most Tekk radios are capable of 9600

> baud operation "out of the box"?

Congratulations. I just sent you the list archive to sort through.

The short answer is: Buy a KS-900, _not_ a KS-960. The KS-900 works

out of the box. There are some very trivial mods, but it will work

without them.

Bruce Perens

-------------------------------------------------------------------------

Ok here is an analysis of two TEKK transceivers that just arrived.

----------------------------------------------------------------

Transceiver #1 - model #KS-900 (black box) SN9697

TX - .8 watts at 7.5volts @ 400ma

2 watts at 10v

All spurious signals down 75 dB or better @7.5v, 80 dB @ 10v

132mv P-P for 3Khz deviation, 1Khz mod freq.

132mv P-P for 2.2 kHz  "    , 5kHz  "    "

For ~ 2.7db rolloff 1/5 kHz

TX distortion not measured, very low.

RX - 3kHz deviation / 1kHz rate

-106.3 dBm / 20 db quieting   (-107dBm = 1 uV)

-110 dBm / 10 dB S+N/N

Distortion 3kHz deviation, -80 dBm

Mod freq      Distortion %

1              2.9

2              3.4

3              6.3

4              9.3

5              7.4

-------------------------------------------------------------------

Trnasceiver #2 - model #KS-900 SN 9701

TX - .9 watts at 7.5 volts @ 400ma

2 watts at 10 volts

No further TX checks run as this is going to be run as a receiver

RX - 3 kHz deviation / 1 kHz rate

-106 dBm / 20 dB quieting

-111 dBm / 10 dB S+N/N

Distortion 3 kHz deviation, -80 dBm

Mod freq.      Distortion %

1               4.75

2               11

3               16.1

4               18.4

5               21.4

----------------------------------------------------------------------

As you can see the RX distortion leaves something to be desired. I

actually ended up using the better RX of the two units for my receiver.

The transmitters looked quite good. Very clean and easy to tune. No

modifications were made to the units for these tests.

We may replace the Murata filters with the flatter units. Has anyone

seen this RX distortion problem? The recovered audio actually starts

to look like a triangle wave at 5Khz or so.

Also for TX output - watch how you measure (and use) this. The

difference between a 1 foot and 4 foot length of RG58 to the

wattmeter results in several tenths of a watt additional loss.

When TEKK specs these things it is power at the connector and

from what I have seen here most units do have 2 watts at the

connector at 10.0 volts. So short (small coax) connections are

a necessity at 440 mHz, that is unless you need the loss.

Doug

----------------------------------------------------------------------