SHOWLOG Program

V4.0

This program has actually several functions included. Originally was intended

to show on screen graphic analysis of the automatic files that Microsats

Satellites creates, such as:

ELTLOGXX   Records single and double bit memory errors

This file is cumulative as many days can be

recorded in this file. It is usually the same

file number, that grows recording new errors.

AL91xxyy   Recording stations and Microsat activity

This file is created one per day, having

on xxyy Month and Day of the log.

WHAT IS NEEDED:

Program had been tested on a PC-XT with CGA (Color Graphics Adapter)

Showlog was made to run on the minimal PC (Could be an XT or a Laptop) having

minumum graphic adapter (CGA). Altough it will be more fast with faster PCs

or Math Coprocessor (tested and used if available) that is no needed to run.

Reason to use minimal PC, was to allow for wide usage and wide audience of

microsats capabilities. This is the reason maps are shown on highest

CGA resolution (640x200) resulting in black & white pictures, and also

to allow easy graphic printing thru DOS Graphics print capabilities.

To run SHOWLOG function you need AL or EL files, these will be used as input

for the program. After download, use PHS to extract it. I.E.

PHS 301.dl /x

You will have after extracting a file either ELTLOGXX.n or AL91xxyy.

This file is input to this program.

You will also need a recent Keplerian data file in AMSAT or NASA format such as:

(numeric data should be on the shown positions, alphabetic data

should be equal to this example)

AMSAT FORMAT:

Satellite: LO-19

Catalog number: 20442

Epoch time:      91043.71839123              Don't worry if there

Element set:     196                         are many satellites on

Inclination:       98.6819 deg               the file, or if there is

RA of node:       124.2620 deg               additional information

Eccentricity:    0.0012688                   as program will use,

Arg of perigee:   167.7080 deg               search and get only

Mean anomaly:     192.4398 deg               required data.

Mean motion:   14.29196610 rev/day

Decay rate:      .00000765 rev/day^2

Epoch rev:            5522

OR

NASA FORMAT:

AO-10

1 14129U 83 58  B 91268.73378291 -.00000087  00000-0  99998-4 0  6987

2 14129  25.7863 122.9103 6060198 279.3182  20.7896  2.05884209 34311

UO-11

1 14781U 84 21  B 91276.11300540  .00002481  00000-0  44618-3 0   740

2 14781  97.8875 316.9668 0011585 183.1405 176.9751 14.67449942405272

Program will read this ASCII file, search for the satellite needed, and

extract keplerian data to calculate satellite position according to recorded

time found in the logs.

A earth map, that's contained on SHOWLOG.MAP, included in this package.

GRAPHICS.COM, a module included in DOS, that will allow you to print

whatever picture this program produces.

On this distribution had been included a Keplerian File (ORBS-278.N)

and example files as AL910925 and ELTLOGXX.3

If you distribute this program include these files to let non-Satellite

users run the program.

----------------------------------------------------------------------------

INSTALLATION

Distribution of SHOWLOG is in two files, one is SHOWLOGP.ZIP contains

the Program (SHOWLOG.EXE when unzipped), the other is SHOWLOGF.ZIP

contains the Files. (You can also receive it on a single zipped file)

Unzip with PKUNZIP these files in a directory (either LUSAT or PACSAT

dir). Then on you can use it calling for the same directory or thru other

directory using C:\PACSAT\SHOWLOG. It is also a recommended option you

prepare a separate directory instead, I.E. \SHOWLOG, and place there all the

files that are on distribution.

----------------------------------------------------------------------------

ERROR LOG ANALYSIS DISPLAY

When you select an ELTLOG for process, program will mark on the map,

where the hit that caused memory error was corrected.

If the error was a single bit, then a cross will be marked, if

double bit then a square will be marked.

On the lower right of the screen, you will see the count of Single error and

Double error, both separated by a slash (/).

Also the memory address and the file (if applicable) are shown.

Display will constantly show start date/time and actual date/time of the

plotted error.

On the lower left of the screen a bar graph will be shown, every vertical

bar representing one day, being its highness according to the amount of

errors, that could be measured along the scale shown.

If the error was a double bit, then you will see a hole on the vertical bar.

When complete log had been read, a file with the name EL91xxyy.PIC will

be written on disk. This file is a copy of the screen, and can be

viewed with the program using the DISPLAY selection.

If you load GRAPHICS.COM before running this program, you can print

a copy of the screen pressing Shift-PrintScreen.

This chart can be used to track where memory errors are corrected, there

is a known geomagnetic perturbation known as 'South Atlantic Anomaly',

that could be easily seen, specially running an UO-14 Log.

You will see also that over populated areas, or areas where the satellite

gets more users, there is also a tendency to show memory errors.

The daily error graph can be used to track solar magnetic storms, or other

external/internal influences that shows over time. Will be interesting to

correlate this graph to the solar activity to see if there is any

correspondence.

Graphs over periods of time could be useful to track overall activity

of the Microsats and tendencies if there are any.

-----------------------------------------------------------------------------

ACTIVITY LOG DISPLAY

When you input to the program an Activity Log, the show Starts.

I will like to express my appreciation and recognition to the gurus

that make the programs that runs on the Microsats, through this program

you can see what a lot of things these boxes can handle simultaneously.

When SHOWLOG analyzes an Activity Log, you will see on the lower right

of the screen the Satellite, AL file under analysis, Date/time, Current

Session and Amount of free memory the satellite has.

On the map, SHOWLOG will place a thin mark where one user connects, if there

are two users then a double wide mark will be placed, if three users then

the satellite path will be more wide, and so on.

So you will see in the map where the satellite gets activity, and by the

wideness of its path the amount of users in that area.

On the lower part of the screen you will see the Call Sign that gets

a connection, as well as its SSID (usually -0).

Also is shown which uplink channel the user is using. Either ^A, ^B, ^C or ^D

These channels corresponds to the uplink frequencies, 145,900, 920, 940 & 960

for the Pacsat or 145,840, 860, 880, 900 for the LUSAT.

Below the user Callsign, you will see which activity is performed, either

DIRs, Downloads, Uploads, etc.

When Downloads or uploads are requested, the program shows which file

had been requested and from which position on that file.

For example D 64E:12432 is saying that file 64E was requested and is

being requested from its position 12432 and on. Same for uploads.

There are four maximum users. When an user LOGs OFF (Disconnect), its

display remains until this Task is being needed for other user.

A fifth user is the Microsat itself, that could be doing internal functions,

such as deleting a file, checking memory, Start/Stop Tasks, etc.

This is shown on a fifth column, when it appears REJECT on this column it is

due all tasks are in use and another user had requested connection.

Also broadcast requests are being handled by the Microsat, and as such

are being shown on this column, as well as the user that requested the

broadcast and on which channel uplinked the request.

When Activity log analysis finishes, program will give a report on

every activity requested to the Satellite, and amount of times requested.

After this screen, if you press enter, you will see a report containing  all

Callsigns (First 77 only, due lack of screen space), that contacted or

broadcasted the Microsat during the Activity log day. The number shown for each

callsign is the amount of LOGON plus Broadcast requests made by that station.

It the station made a Broadcast, it will be marked with a tiny point close

to the number on that Callsign number of Logons.

Program will create an AL91xxyy.PIC containing map and BBS statistics and

a file AU91xxyy.PIC containing map and users Callsigns.

You can print either report pressing Alt-PrtSc.

You can append several ALOG files for a continued activity show if required.

Program can be slowed pressing -/+ Keys. Speed will be marked accordingly.

Maximum speed is 10.

Pressing/holding Space key will flash Satellite position on screen.

If you like fancy noises, then press Y and will hear the Microsat Symphony.

When seen, your call will be rewarded with a special sound

by the Microsat SHOWLOG.

Any suggestions, critics or insults are welcome.

ENJOY!

Best 73s, LU7ABF, Pedro Converso.  AMSAT Argentina Member.

-------------------------------------------------------------------------------

Changes in V2.0  06/91

- Added remembering of last run data entries. (File SHOWLOG.DAT creation)

(W6SHP, K4EVH Suggestion, thanks Howard & Ken)

- Added dynamic selection to plot full satellite data track. (Show/Hide)

(W6SHP, DF5DP Suggestion, thanks Howard & Norbert)

- Added selection to write/not write LOGs files pictures.

(DF5DP Suggestion, thanks Norbert)

- Correction of erratic incorrect screen placing of Broadcast requests.

(W6SHP suggestion, thanks Howard)

- Dinamically positioning stations report at end of program.

(LU4ENQ Suggestion, Gracias Carlos)

- Added Help Function, available pressing F1 Key. Brings a help file

(SHOWLOG.HLP), that can be modified or written in National Language.

(W6SHP Suggestion, Thanks Howard)

- Satellite position can be made more visible pressing or holding Space key.

(Actual position will be flashing)  (W6SHP Suggestion, Thanks Howard)

- Added graphic Image of a Microsat over an earth background shown

during presentation. Can also be recall by F2 Key. Image on SHOWLOG.PIC file.

- Added for educational usage a picture with inside distribution of a

Microsat, and another picture with typical Microsat Earth Station.

(Images on SHOWLOG0.PIC (Showlog Station) and SHOWLOG1.PIC (Microsat)).

- Added possibility to show up to ten presentation pictures.

- Added Color and Pallete (c/p) selection on Presentation Screens.

- Increased stations report for up to 91 stations per Activity Log.

- Added recognition of HIGHTIME exchange commands during DIR requests,

showing during analysis time of user HIGHTIME when requesting DIRs.

- Added recognition of new, not yet used commands to/from Satellite.

- Improved speed of Orbit,plotting and Log File calculations.

Want also thanks fine reports of EI6EH (Tom), ZL1TRE (Mark), KB5MU (Paul),

N8AM (Tom), W0SL (Roy), LU7XAC (Nestor), and WB6LLO (Dave).

-------------------------------------------------------------------------------

Changes in V3.0  08/91

- Added recognition of UoSAT Oscar-14 commands.

- Channel Usage now more accurate, taking only into account Alog entries

that contains Uplink Channel.

- Added Uplink channel show on Logoff and other commands containing Uplink.

- Giving increase of activity extended final report up to 200 stations.

- Giving added two new function Keys, now F Key Function shows meaning

of F-Keys.

- Added Simulation Function (F3), using SHOWLOG.SIM File to simulate

actual activity of AMSAT programs Microsatters use. Now you can

recall Microsat at any time to show what you do when actual Satellite

is passing. (SHOWLOG.SIM formatting commands explained in Help, allows

to create any simulation required)

- Added Noise music to simulate Microsat during emulation, selected by

left/right cursor keys.

- Added Directory Functions (F4). This function Reads your PG.DIR in

current directory, and formats it on screen, allowing paging through

all directory, seeing of Keywords, dates, etc. (Maximum files 640)

- Added View file function. Thru Directory you can highlight select any file.

SHOWLOG will try to locate .dl .dlx .ul .pdl files and do an

online extraction of the file to show it on screen. You can see it

also in Hex form thru "H" selection.

- Added View File function in Hexadecimal to view non-ASCII files,

and page through them in hexadecimal addresses.

- Added exit to your Editor to prepare files.

Will return to the point where you leave when coming back from Editor.

- Editor Selection recorded on SHOWLOG.DAT avoids reentering it.

- Added exit to DOS to perform any DOS function or call any other program.

Will return to the point where you leave when coming back from DOS.

- Split in two files for distribution (SHOWLOGP, Pgm and SHOWLOGF, files)

-------------------------------------------------------------------------------

Changes in V4.0   11/91

- Completely reshaped program to allow working through Menues.

- Menu friendly function allow to place your data and select switches

the program uses.  Also your location data, Name, etc can be placed

on menu and reused for future runs.

- Added to save all your last setting on a file for next run usage

(SHOWLOG.MNU) (Previous version used SHOWLOG.DAT)

- Every menu selection now explains shortly what is it for on lower

part of Menu Screen.(Included in SHOWLOG.MNU could be adapted to

your national language).

- Speed up of calculus through preseted time/orbits calculations, will

speed showlog analysis as well as predictions.

- Added reading and recognition of NASA Format for Keplerians, now both

AMSAT and NASA straight Ascii formats could be used.

- New Function: Select Sat

Allow you to see Keplerian data for any Satellite, as well as an

estimated launch time based on Keplerians, and on which position

over earth is any Satellite. Sun included as a Satellite.

(Who ever say Ptolomeus was wrong?, Doesn't Sun orbits earth?)

- New Function: Predictions (or Tracking)

This function allow you to move at your wish any satellite over earth

and over time. Will show what Satellite sees from earth (Its coverage)

- Added Local timezone. All times are given in your watch local time,

simplifying your schedules to listen to the birds.

- Added schedule function. Calculates next usable passes for any satellite

for your location.

- Added Tracking by minute function. You can see where to point your antenna

on Azimuth and Elevation at up to 24 timesteps within the pass.

- Added Show Pass on map as well as Time, Azimuth and Elevation every

minute or in 24 steps for all the pass.

- Added browse of the twelve calculated passes showing track thru Enter.

- Added Satellite tracking plot. You can see differences in orbits for

sun-synchronous Satellites, Elliptical Orbits, Reverse Orbits, etc.

- Added Short description of frequencies any Satellite uses (in SHOWLOG.SAT)

- Added recognition of Geo-Synchronous Satellites, and where to point your

antenna in Azimuth and Elevation.

- Added Sun's Keplerians hardwired into program. Allows to see sunrise and

sunset for several days. You can see winter and summer coming to you.

Also Sun's position on any day will be shown in Azimuth/Elevation on 24

steps during the day.

- Added Presentation capabilities, through continuous show of pictures.

- Added Selection for a cross marked on every Satellite position on Showlog.

- Added Selection for full track plot on Menu.

- New Function: Telemetry

Will allow to call inline your Telemetry program or the one included

(TBR by LU4AEY, Claudio). Telemetry can be recorded by PB Kiss function

or by TBR or any other program and now analyzed and shown within Showlog.

- Enhanced Help now includes more details for all the functions.

- Added ESC key recognition on any function. This will return you to Menu.

- Executable programs now autoexpands using LZ. Fast load and less

disk Space occupied. All programs/files fit in a single 360 KB diskette.

73s, Pedro, LU7ABF. p/AMSAT Argentina