BSQ.DOC v2.0          2/20/86 W1GOH

BSQ is a program designed to allow transfer of arbitrary binary

files via packet radio.  It converts a binary file to printable

ASCII characters, performing data compression where possible;

the result may be transmitted (in "conversational" mode) and

reconverted (again using BSQ) to binary at the other end.

BSQ is available from the W1GOH BBS both in source and binary form,

the latter running on IBM PCs and other MSDOS machines.  It is

implemented in "vanilla" C, and should be easily portable to CPM

or other environments.

*** NB: Transfer of large files on a busy packet channel is both

*** slow and somewhat antisocial.  Try to confine big transfers

*** to lightly-used times and/or frequencies!

USAGE:

bsq <options> file.ext [outname]

reads binary file.ext, writes file.bsq (or outname if specified)

bsq <options> file.bsq [outname]

reads file.bsq, writes file whose name is same as original

(or outname, if specified).

BSQ default output is always in the current directory. WARNING: existing

files whose name conflicts with the output file name are overwritten.

<options> may include:

-b   Bootstrap mode.  Does simple bit-stuffing, no data

compression.  Output file suitable for simple BASIC pgm

BSQBOOT.BAS.

-o     Old mode. Used to specify creation of .BSQ files compatible

with earlier versions of BSQ.

You may "bootstrap" BSQ to your machine, over the air, via either

of two ways:

(1) If you have a C compiler, you may download BSQ/BSQ.C and

compile it.  Careful... BSQ.C is 23K bytes!

(2) If you have an MSDOS machine and Microsoft BASIC or equivalent,

you may download BSQ/BSQBOOT.BAS along with BSQIBM.BSQ (16K!),

which must be renamed BSQ.BSQ on your local (MS-DOS) machine.

BSQBOOT.BAS reads BSQ.BSQ, and writes the executable file

BSQ.COM.  This process runs VERY slowly (about 10 minutes

on an IBM PC/XT!).