Hypertext Transfer Protocol (HTTP)        Tim Berners-Lee, CERN

Internet Draft

Expires 5 May 1994                                   5 Nov 1993

Hypertext Transfer Protocol (HTTP)

A Stateless Search, Retrieve and Manipulation Protocol

Status of this memo

This document is an Internet Draft. Internet Drafts are working

documents of the Internet Engineering Task Force (IETF), its Areas,

and its Working Groups.  Note that other groups may also distribute

working documents as Internet Drafts.

Internet Drafts are working documents valid for a maximum of six

months. Internet Drafts may be updated, replaced, or  obsoleted by

other documents at any time.  It is not appropriate to use Internet

Drafts as reference material or to cite them other than as a

"working draft" or "work in progress".

This document is a DRAFT specification of a protocol in use on the

internet and to be proposed as an Internet standard.   Discussion

of this protocol takes place on the www-talk@info.cern.ch mailing

list -- to subscribe mail to www-talk-request@info.cern.ch.

Distribution of this memo is unlimited.

Abstract

HTTP is a protocol with the lightness and speed necessary for a

distributed collaborative hypermedia information system.  It is a

generic stateless object-oriented protocol, which may be used for

many similar tasks such as name servers, and distributed

object-oriented systems, by extending the commands, or "methods",

used. A feature if HTTP is the negotiation of data representation,

allowing systems to be built independently of  the development of

new advanced representations.

Note: This specification

This HTTP protocol is an upgrade on the original protocol as

implemented in the earliest WWW releases.  It is back-compatible

with that more limited protocol.

This specification includes the following parts:

The Request

T. Berners-Lee                                                       1

Methods

A list of headers in the request message

The response

Status codes

A list of headers on any object transmitted

The content of any object content transmitted

Format negotiation algorithm

The HTTP Registration Authority

Security Considerations

Unresolved points

References

The following notes form recommended practice not part of the

specification:

Servers tolerating clients

Clients tolerating servers

Purpose

When many sources of networked information are available to a

reader, and when a discipline of reference between different

sources exists,  it is possible to rapidly follow references

between units of information which are provided at different remote

locations.   As response times should ideally be of the order of

100ms in, for example, a hypertext jump, this requires a fast,

stateless, information retrieval protocol.

Practical information systems require more functionality than

simple retrieval, including search, front-end update and

annotation. This protocol allows an open-ended set of methods to be

used. It builds on the discipline of reference provided by the

Universal Resource Identifier (URI) as a name (URN, RFCxxxx)  or

address (URL, RFCxxxx) allows the object of the method to be

specified.

Reference is made to the Multipurpose Internet Mail Extensions

(MIME, RFC1341) which are used to allow objects to be transmitted

in an open variety of representations.

Overall operation

T. Berners-Lee                                                       2

On the internet, the communication takes place over a TCP/IP

connection. This does not preclude this protocol being implemented

over any other protocol on the internet or other networks.   In

these cases, the mapping of the HTTP request and response

structures onto the transport data units of  the protocol  in

question is outside the scope of this specification. It should not

however be at all complicated.

The protocol is basically stateless, a transaction consisting of

Connection              The establishment of a connection by the

client to the server - when using TCP/IP

port 80 is the well-known port, but other

non-reserverd ports may be specified in the

URL;

Request                 The sending, by the client, of a request

message to the server;

Response                The sending, by the server, of a response to

the client;

Close                   The closing of the connection by either both

parties.

The format of the request and response parts is defined in this

specification. Whilst header information defined in this

specification is sent in ISO Latin-1 character set in CRLF

terminated lines, object transmission in binary is possible.

Character sets

In all cases in HTTP where RFC822 characters are allowed, these may

be extended to use the full ISO Latin 1 character set.  8-bit

transmission is always used.

tableofcontents

REQUEST

The request is sent with a first line containing the method to be

applied to the object requested, the identifier of the object, and

the protocol version in use, followed by further information

encoded in the RFC822 header style. The format of the request is:

Request           =     SimpleRequest | FullRequest

SimpleRequest     =     GET <uri> CrLf

FullRequest       =     Method UR ProtocolVersion CrLf

[*<HTRQ Header>]

[<CrLf> <data>]

T. Berners-Lee                                                       3

<Method>          =     <InitialAlpha>

ProtocolVersion   =     HTTP/V1.0

uri               =     <as defined in URL spec>

<HTRQ Header>     =     <Fieldname> : <Value> <CrLf>

<data>            =      MIME-conforming-message

The URI is the Uniform Resource Locator (URL) as defined in the

specification, or may be (when it is defined) a Uniform Resource

Name (URN)  when a specification for this is settled, for servers

which support URN resolution.

Unless the server is being used as a gateway, a partial URL shall

be given with the assuptions of the protocol (HTTP:) and server

(the server) being obvious.

The URI should be encoded using the escaping scheme described in

the URL specification to a level such that (at least) spaces and

control characters  (decimal 0-31 and 128-159) do not appear

unesacaped.

Note. The rest of an HTTP url after the host name and optional port

number is completely opaque to the client: The client may make no

deductions about the object from its URL.

Protocol Version

The Protocol/Version field defines the format of the rest of the

request.. At the moment only HTRQ is defined .

If the protocol version is not specified, the server assumes that

the browser uses HTTP version 0.9.

Uniform Resource Identifier

This is a string identifying the object.  It contains no blanks. It

may be a Uniform Resource Locator [ URL ] defining the address of

an object as described in RFCxxxx, or it may be a representation of

the name of an object  (URN, Universal Resource Name) where that

object has been registered in some name space.  At the time of

writing, no suitable naming system exists, but this protocol will

accept such names so long as they are  distinguishable from the

existing URL name spaces.

Methods

Method field indicates the method to be performed on the object

identified by the URL.  More details are with the list of  method

names below .

T. Berners-Lee                                                       4

Request Headers

These are RFC822 format headers with special field names given in

the list below ,  as well as any other HTTP object headers or  MIME

headers.

Object Body

The content of an object is sent (depending on the method) with the

request and/or the reply.

Methods

The Method field in HTTP indicates the method to be performed on

the object identified by the URL.  The method GET below is always

supported, The list of other methods acceptable by the object are

returned in response to either of these two requests.

This list may be extended from time to time by a process of

registration with the design authority.  Method names are case

sensitive. Currently specified methods are as follows:

GET                     means retrieve whatever data is identified

by the URI, so where the URI refers to a

data-producing process, or a script which can

be run by such a process, it is this data

which will be returned, and not the source

text of the script or process. Also used for

searches .

HEAD                    is the same as GET but returns only HTTP

headers and no document body.

CHECKOUT                Similar to GET but locks the object against

update by other people. The lock may be

broken by a higher authority or on timeout:

in this case a future CHECKIN will fail.  (

Phase out? )

SHOWMETHOD              Returns a description (perhaps a form) for a

given method when applied to the given

object. The method name is specified in a

For-Method: field. (TBS)

PUT                     specifies that the data in the body section

is to be stored under the supplied URL.  The

URL must already exist.  The new contenst of

the document are the data part of the

request. POST and REPLY should be used for

creating new documents.

DELETE                  Requests that the server delete the

information corresponding to the given URL.

T. Berners-Lee                                                       5

After a successfull DELETE method, the URL

becomes invalid for any future methods.

POST                    Creates a new object linked to the specified

object. The message-id field of the new

object may be set by the client or else will

be given by the server. A URL will be

allocated by the server and returned to the

client. The new document is the data part of

the request.  It is considered to be

subordinate to the specified object, in the

way that a file is subordinate to a directory

containing it, or a news article is

subordinate to a newsgroup to which it is

posted.

LINK                    Links an existing object to the specified

object.

UNLINK                  Removes link (or other meta-) information

from an object.

CHECKIN                 Similar to PUT, but releases the lock set on

the object.  Fails if no lock has been set by

CHECKOUT. Suggestion : phase out this

(rcs-like) model in favor of the PUT

(cvs-like, non-locking) model of code

management.

TEXTSEARCH              The object may be queried with a text

string.  The search form of the GET method is

used to query the object.

SPACEJUMP               The object  will accept a query whose terms

are the cooridnates of a point within the

object. The method is implemented using GET

with a derived URL .

SEARCH                  Proposed only.  The index (etc) identified

by the URL is to be searched for something

matching in some sense the enclosed message.

How does the client know what message fromats

are acceptable to the server? (Suggestion of

Fred Williams)

(Some of these methods require more detailed specification)

Note: case sensitivity of method names

Although lack of case sensitivity in methos names would be a

tolerant approach with a limited method set,  we require the

extensibility of HTTP to cover an arbitrary underlying object

system.  In such a system, method names may be case sensitive, and

T. Berners-Lee                                                       6

so we must preserve case in HTTP.

GET

A representation of the object is transferred to the client.

Some URIs refer to specific variants of an object, and some refer

to objects with many variants. In the latter case, the

representations, encodings, and languages acceptable may be

specified in the header request fields, and may affect the

particular value which is returned.

Other possible replies allow  a set of URIs to be returned to the

client, who may use them to retrieve the object.  This allows name

servers to be implemented using HTTP, and also forwarding address

to be given when objects have been moved.

Annotation replies

Some servers keep 'parallel webs" -- separate stores of information

about other server's objects.  Typically this incldues annotation

links.   This information may be retrieved using the GET method,

but a special reply is returned.

Searching using GET

When the TEXTSEARCH or SPACEJUMP methods are supported, their

implementation is in fact by means of the GET method, by

constructing a derived URL.

The URL used with GET is the URL of the object, suffixed by a "?"

character and the text to be searched. If the object being searched

is in fact itself the result of a search (ie the URL contains a

"?"), then those search terms are first stripped off, so the search

is performed on the original object.

In the URL, keywords are seperated with plus signs ("+").  Real

spaces, plus signs, and other illegal characters [who defines

illegal characters?] are represented as hexadecimal ASCII escape

sequences (%##, e.g., space = %20, plus = %2B)

Browsers accepting text input should define keywords as seperated

by spaces (and therefore map them to plus signs). More advanced

browsers may allow separate keyword input and, therefore, a finer

level of control over the content without users needing to

understand the underlying mechanisms (e.g., that they must use %20

to get a real space).

Examples

The following is a request on an object supporting TEXTSEARCH:

T. Berners-Lee                                                       7

GET   /indexes/botany?annual+plants     HTTP/1.0

This request on an object supporting SPACEJUMP selects point

(0.2,3.8) within a map:

GET   /maps/uk/dorset/winspit/0.2+3.8   HTTP/1.0

LINK

The link method of HTTP adds meta information (Object header

information) to an object, without touching the object's content.

For example, it requests the creation of a link from the specified

object to another object.

The request is followed by a set of object headers which are to be

added.

In cases in which a new header is added and the semantics of the

header do not allow it to coexist with a similar header, then the

previous header is deleted.  For example, an object may only have

one title, so specifying a title overwrites and preexsiting title.

Link Types

The link type unless void is specified in the WWW-Link object field

for each link.  If not specified, void is assumed.

Unresolved points

As this is generalised to allow any metainformation to be added,  a

better name might be DESCRIBE or attribute (as a verb). I don't

like "describe" as it does not suggest alteration.

"Bestow"/"Rescind" might be better. For example, one could bestow a

title or author on something previously title-less.  We are looking

at a general data model behind here a little like a relational

database.  This function adds records.   "Set" and "Reset"?

Related Methods

LINK is like POST except that

no storage is requested for the destination object, and

the destination is not assumed to be subordinate to the

specified object.

See also: UNLINK.

Tim BL

POST

T. Berners-Lee                                                       8

This method of HTTP creates a new object linked to and subordinate

to the specified object.   The content of the new object is

enclosed as the body of the request.

The POST method is desiged to allow a uniform function to cover

Annotation existing documents;

Posting a message to a bulletin board topic, newsgroup, mailing

list, or similar group of articles;

Adding a file to a directory;

Extending a document during authorship.

The client may not assume any postconditions of the method in terms

of web topology. For example, if a POST is accepted, the effect may

be delayed or overruled by human moderation, batch processing, etc.

The client should not be surprised if a link is not immediately

(or never) created.

However, the semantics are a request for a link to be made from the

object whose URL is quoted to the new, enclosed, object.

If no URL for the NEW object is given by the client, the server is

requested to see to the storage of the new object.  That is, the

server does not have to store it but will have to return a URL be

which it can later be retrieved.  The semantics of this method

(currently) imply nothing of any undertakings by the server to

maintain the availability of the object.

If the client gives a URL, then the server is not obliged to store

the object, but may take a copy and may in that case issue a new

URL.

Return object headers

The method shall return a set (possibly empty) of object headers

for the newly posted object.  If a URL has been assigned by the

server, then that may be included.  Similarly, if a URN has been

assigned, then that shall be returned. Other things which may be

returned include for example the expiry-date if any. The server may

return the entire metadata for the object (as in the HEAD command),

or a subset of it.

The object body shall not be returned, so the transaction shall end

with the blank line terminating the headers.

Link type

The link type may be specified by explicitly giving a (reverse)

link in the object header of the linked object.  If a link or links

between the two objects are present in those headers, then that

T. Berners-Lee                                                       9

link is used. If no such link is specified, then the server should

generate a link. The link type in this case is determined by the

server.  The server may perform other operations as a result of the

new object being added: lists and indexes might be updated, for

example.

Submission

When articles are submitted, the analogy of being addeed to a body

of knowledge by being linked is close. When a form is submitted,

this can be done with POST, though in this case side-effects will

be expected.

(Should submission for action have a different method -- see

showmethod -- or should it be just POST for simplicity?  When

interfacing with other systems such as bews and mail, the

distinction is not made as the system does not have the ability to

distinguish different methods. We now have a possibility of making

a separate action, though.)

Unresolved points

The client has no way of knowing what data formats the server is

prepared to accept. This may not be a serious problem, but it may

be if the server has some restrictions.  This applies to all

submissions of object content from client to server, for all

methods.

Related methods.

When a new URL has been returned by the server, it may in general

(typically, but not necessarily) be usable as the argument of

DELETE , GET , PUT , etc, methods.

To make a link between two existing objects, see LINK.

Tim BL

SEARCH

Some correspondance about SEARCH:

Q

How did the client know that the server could search for tex?  Maybe t

his server can only serach for text and GIF patterns, even though it g

ave out tex.  How can the client know that a GIF search is possible?

Answer

I assume the following has happened:

client requests with ACCEPT: text/x-TeX

T. Berners-Lee                                                       10

If the server, which it should if possible, replies with the doc

requested in ``TeX''.

Therefore a server can obviously do an ``internal representation'' to

``TeX'' conversion.

Since the server responded in a particular representation (ie TeX)

The client should be able to search in ``TeX'' provide the ``SEARCH''

method is in the required ALLOWED: section of the response.

The server should only respond with ALLOWED: SEARCH ... if it can do a

Tex to ``internal representation'' conversion for searching.

Fred Williams fwilliam@ccs.carleton.ca

SHOWMETHOD

When an object can support more operations than are defined in this

specification, SHOWMETHOD allows a client to understand the

interface to that operation sufficiently to allow the user to

perform it interactively.

Required parameter field

For-Method:             This filed contains only the method name

about which the client is inquiring.

Preconditions

The methodname spacified in the For-Method field must have been

previously issued in a  "Allowed:" field returned with the given

object.

The client should specify an Accept: field which includes at least

one form langauge it it wants to be able to interpret the result.

Postcondidtion

SHOWMETHOD returns, if possible, a form in a representation

acceptable to the client.  This form will contain instructions for

ordering the operation, and fields for the parameters.

A suitable language for the form might be HTTP2, but any language

may be used which is acceptable to the client.

SPACEJUMP

This method is similar to the TEXTSEARCH method, but instead of the

search criterion being a text string, it is a set of coordinates

defining a point within the image.  The semantics of the operation

are not defined here.  Typically, the user clicks on a point within

the image with a mouse or other pointing device.

T. Berners-Lee                                                       11

Two or more coordinates are supplied, in the order x, y z, t.  All

coordinates are scaled so that 0 represents the bottom left hand

point and 1.0 represents the top right hand point.

The z access direction follows the normal right-hand rule, that is

extends toward the viewer when the x and y axes are flat as in the

normal two-dimensional representation.

In the case of a time-occupying object, 0 represents the starting

instance, and 1.0 represents the finishing instant.

The method is implemented using GET with a derived URL .

TEXTSEARCH

This is a simple form of search. The text is assumed to derive from

the requesting user, and is in no special format.

The exact algorithm to be applied is not defined in this

specification, but techniques such as vocabulary proximity matching

between the request data portion and the contents or titles of

documents, keyword matching, stemming, and the use of a thesaurus

are quite appropriate.

Whilst this method name is given as a flag to specify that the

function is available, the search form of the GET method is in fact

used to query the object.

UNLINK

This method deletes metainformation about an object.  The request

contains object geaders which are to be removed.  Only headers

exactly matching the headers given are removed.

Obviously the operation may be used for unlinking objects. It may

also be used for removing other metainformation such as object

title, expiry date, etc.

Related methods

See LINK.

Tim BL

HTTP Request fields

These header lines are sent by the client in a HTTP protocol

transaction. All lines are  RFC822 format headers. The list of

headers is terminated by an empty line.

FROM:

In Internet mail format, this gives the name of the requesting

T. Berners-Lee                                                       12

user. This field may be used for logging purposes and an insecure

form of access protection.  The interpretation of this field is

that the request is being performed on behalf of the person given,

who accepts responsability for the method performed.

The Internet mail address in this field does not have to correspond

to the internet host which issued the request. (For example, when a

request is passed through a gateway, then the original issuer's

address should be used).

The mail address should, if possible, be a valid mail address,

whether or not it is in fact an internet mail address or the

internet mail representation of an address on some other mail

system.

ACCEPT:

This field contains a comma-separated list of representation

schemes (Content-Type  metainformation values) which will be

accepted in the response to this request.

The set given may of course vary from request to request from the

same user.

This field may be wrapped onto several lines according to RCFC822,

and also more than one occurence of the field is allowed with the

signifiance being the same as if all the entries has been in one

field. The format of each entry in the list is (/ meaning "or")

<field>  =    Accept: <entry> *[ ; <entry> ]

<entry>  =    <content type> *[ , <param> ]

<param>  =    <attr> = <float>

<attr>   =    q / mxs / mxb

<float>  =    <ANSI-C floating point text represntation>

See the appendix on the negotiation algorithm as a function and

penalty model.

If no Accept: field is present, then it is assumed that text/plain

and text/html are accepted.

Example

Accept: text/plain; text/html

Accept: text/x-dvi, q=.8, mxb=100000, mxt=5.0; text/x-

c

Wildcards

In order to save time, and also allow clients to receive content

types of which they may not be aware, an asterisk  "*" may be used

T. Berners-Lee                                                       13

in place of either the second half of the content-type value, or

both halves.  This only applies to the Accept: filed, and not to

the content-type field of course.

Example

Accept:  *.*, q=0.1

Accept:  audio/*, q=0.2

Accept:  audio/basic q=1

may be interpreted as "if you have basic audio, send it; otherwise

send me some other audio, or failing that,  just give me what

you've got."

ACCEPT-ENCODING:

Similar to Accept,  but lists the Content-Encoding types which are

acceptable in the response.

<field>  =    Accept-Encoding: <entry> *[ , <entry> ]

<entry>  =    <content transfer encoding> *[ , <param> ]

Example

Accept-Encoding: x-compress; x-zip

ACCEPT-LANGUAGE:

Similar to Accept, but lists the Language values which are

preferable in the response.  A response in an unspecifies language

is not illegal. See also: Language .

Language coding TBS.  (ISO standard xxxx)

USER-AGENT:

This line if present gives the software program used by the

original client. This is for statistical purposes and the tracing

of protocol violations. It should be included.  The first white

space delimited word must be the software product name, with an

optional slash and version designator. Other products which form

part of the user agent may be put as separate words.

<field>   =   User-Agent: <product>+

<product> =   <word> [/<version>]

<version> =   <word>

Example:

UserAgent:  LII-Cello/1.0  libwww/2.5

REFERER:

T. Berners-Lee                                                       14

This optional header field allows the client to specify, for the

server's benefit, the address ( URI ) of the document  (or element

within the document) from which the URI in the request was

obtained.

This allows a server to generate lists of back-links to documents,

for interest, logging, etc.  It allows bad links to be traced for

maintenance.

If a partial URI is given, then it should be parsed relative to the

URI of the object of the request.

Example:

Referer: http://info.cern.ch/hypertext/DataSources/Over

view.html

AUTHORIZATION:

If this line is present it contains authorization information. The

format is To Be Specified (TBS). The format of this field is in

extensible form. The first word is a specification of the

authorisation system in use.

Proposals have been as follows: (see the specification for current

one implemented by AL Sep 1993)

User/Password scheme

Authorization:  user  fred:mypassword

The scheme name is "user".   The second word is a user name

(typically derived from a USER environment variable or prompted

for), with an optional password separated by a colon (as in the URL

syntax for FTP).  Without a password, this povides very low level

security.  With the password, it provides a low-level security as

used by unmodified FTP, Telnet, etc.

Kerberos

Authorization:  kerberos  kerberosauthenticationsparam

eters

The format of the kerberosauthenticationsparameters is to be

specified.

CHARGETO:

This line if present contains account information for the costs of

the application of the method requested. The format is TBS. The

format of this field must be in extensible form.  The first word

starts with a specification of the namespace in which the account

is . (This is similar to extensible URL definition.) No namespaces

T. Berners-Lee                                                       15

are currently defined. Namespaces will be registered with the

registration authority .

The format of the rest of the line is a function of the charging

system, but it is recommended that this include a maximum cost

whose payment is authorized by the client for this transaction, and

a cost unit.

Note: Server tolerance of bad clients

Whilst it is seen appropriate for testing parsers to check full

conformance to this specification, it is recommended that

operational parsers be tolerant of deviations.

In particular, lines should be regarded as terminated by the Line

Feed, and the preceeding Carriage Return character ignored.

Any HTTP Header Field Name which is not recognised should be

ignored in operational parsers.

It is recommended that servers use URIs free of "variant"

characters whose representation differs in some of the national

variant character sets, punctuation characters, and spaces.  This

will make URIs easier to handle by humans when the need (such as

debugging, or transmission through non hypertext systems) arises.

RESPONSE

The response from the server shall start with the following syntax

(See also:  note on client tolerance ):

<status line>   ::=    <http version>  <status code>  <reason line>

<CrLf>

<http version>  ::=    3*<digit>

<status code>   ::=    3*<digit>

<digit>         ::=    0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9

<reason line> ::=   * <printable>

<http version>         identifies the HyperText Transfer Protocol

version being used by the server.  For the

version described by this document version it

is  "HTTP/1.0" (without the quotes).

< status code >         gives the coded results of the attempt to

understand and satisfy the request. A three

digit ASCII decimal number.

<reason string>         gives an explanation for a human reader,

except where noted for particular status

codes.

Fields on the status line are delimited by a single blank (parsers

T. Berners-Lee                                                       16

should accept any amount of white space). The possible values of

the status code are listed below .

Response headers

The headers on returned objects those RFC822 format headers listed

as object headers , as well as any MIME conforming headers, notably

the Content-Type field.  Note that this specification doesnot

define any headers particular to the response which are not also

apropriate to any transmission of an object with a request.

Response data

Additional information may follow, in the format of a MIME message

body. The significance of the data depends on the status code.

The Content-Type used for the data may be any Content-Type which

the client has expressed his ability to accept, or text/plain, or

text/html. That is, one can always assume that the client can

handle text/plain and text/html.

Status codes

The values of the numeric status code to HTTP requests are as

follows. The data sections of messages Error, Forward and

redirection responses may be used to contain human-readable

diagnostic information.

SUCCESS 2XX

These codes indicate success. The body section if present is the

object returned by the request. It is a MIME format object. It is

in MIME format, and may only be in text/plain, text/html or one fo

the formats specified as acceptable in the request.

OK 200

The request was fulfilled.

CREATED 201

Following a POST command, this indicates success, but the textual

part of the response line indicates the URI by which the newly

created document should be known.

Accepted 202

The request has been accepted for processing, but the processing

has not been completed.  The request may or may not eventually be

acted upon, as it may be disallowed when processing actually takes

place. there is no facility for status returns from asynchronous

operations such as this.

T. Berners-Lee                                                       17

Partial Information 203

When received in the response to a GET command, this indicates that

the returned metainformation is not a definitive set of the object

from a server with a copy of the object, but is from a private

overlaid web. This may include annotation information about the

object, for example

ERROR  4XX, 5XX

The 4xx codes are intended for cases in which the client seems to

have erred, and the 5xx codes for the cases in which the server is

aware that the server has erred.  It is impossible to distinguish

these cases in general, so the difference is only informational.

The body section may contain a document describing the error in

human readable form. The document is in MIME format, and may only

be in text/plain, text/html or one for the formats specified as

acceptable in the request.

Bad request 400

The request had bad syntax or was inherently impossible to be

satisfied.

Unauthorized 401

The parameter to this message gives a specification of

authorization schemes which are acceptable.  The client should

retry the request with a suitable Authorization header.

PaymentRequired 402

The parameter to this message gives a specification of charging

schemes acceptable.  The client may retry the request with a

suitable ChargeTo header.

Forbidden 403

The request is for something forbidden. Authorization will not

help.

Not found 404

The server has not found anything matching the URI given

Internal Error 500

The server encountered an unexpected condition which prevented it

from fulfilling the request.

Not implemented 501

T. Berners-Lee                                                       18

The server does not support the facility required.

REDIRECTION 3XX

The codes in this section indicate action to be taken (normally

automatically) by the client in order to fulfill the request.

Moved 301

The data requested has been assigned a new URI, the change is

permanent. (N.B. this is an optimisation, which must,

pragmatically, be included in this definition.  Browsers with link

editing capabiliy should automatically relink to the new reference,

where possible)

The response contains one or more header lines of the form

URI: <url> String CrLf

Which specify alternative addresses for the object in question.

The String is an optional comment field. If the response is to

indicate a set of variants which each correspond to the requested

URI, then the multipart/alternative wrapping may be used to

distinguish different sets

Found 302

The data requested actually resides under a different URL, however,

the redirection may be altered on occasion (when making links to

these kinds of document, the browser should default to using the

Udi of the redirection document, but have the option of linking to

the final document) as for "Forward".

The response format is the same as for Moved .

Method 303

Method: <method> <url>

body-section

Like the found response, this suggests that the client go try

another network address.  In this case, a different method may be

used too, rather than GET.

The body-section contains the parameters to be used for the method.

This allows a document to be a pointer to a complex query

operation.

@@TBS in more detail.

The body may be preceded by the following additional fields as

listed .

T. Berners-Lee                                                       19

Object MetaInformation

The header fields given with or in relation to objects in  HTTP are

as follows.   All are optional. These headers specify

metainformation: that is, information about the object, not the

information which is contained in the object.

There is no reason for limiting these fields to HTTP use, as any

other system which requires metainformation is encouraged to use

them.

The order of header lines withing the HTTP header has no

significance. However, those fields which are not MIME fields

should occur before the MIME fields, so that the MIME fields and

following form a valid MIME document. This is not mandatory.

Any header fields which are not understood should be ignored.

(TBS in more detail)

ALLOWED:   *METHOD

Lists the set of requests which the requesting user is allowed to

issue for this URL.  If this header line is omitted, the default

allowed methods are "GET  HEAD"

Example of use:

Allow: GET HEAD PUT

PUBLIC: *METHOD

As "Allow" but lists those requests which anyone may use.   If

omitted, the default is "GET" only.

Example of use:

Public: GET HEAD TEXTSEARCH

CONTENT-LENGTH: INT

Implies that the body is binary and should be read directly from

the communications link, without parsing lines, etc.   When the

data is part of the request, prevents the escaping and de-escaping

of the termination sequence .

@@@ This should be part of the MIME header, as it applies to any

binary encoded part. Note HTML is the first internet  protocol to

allow MIME "binary" encoding. In MIME, the use of Content-Length is

currently allowed only for external messages.

CONTENT-TYPE:

T. Berners-Lee                                                       20

As defined in MIME, except where noted here.

Extra non-MIME types

It is reasonable to put strict limits on transfer formats for mail,

where there is no guarantee that the receiver will understand a

weird format. However, in HTTP one knows that the receiver will be

able to receive it because it will have been sent in the Accept:

field.  There is therefore a lot to be gained from a very complete

registry of well-defined types for HTTP which may nevertheless not

be recommended for mail. In this case, the content-type list for

HTTP may be a superset of the MIME list.

The x- convention for experimental types is of course still

available as well.

Type parameters

Parameters on the content type are extremely useful for describing

resolutions, colour depths, etc. They will allow a client to

specify in the Accept: field the resolution of its device. This may

allow the server to economise greatly on transmission time by

reducing the resultion of an image, for example.

These parameters are to be specified when types are registered..

@@ TBS.

Multipart types

MIME provides for a number of "multipart" types.  These are

encapsulations of several body parts in the one message.  In HTTP,

Multipart types may be returned on the condition that the client

has indicated acceptability (using Accept:)of the multipart type

and also of the content types of each consitutent body part.

The body parts (unlike in MIME) MAY contain HTTP metainformation

header fields which ARE significant.

Multipart/alternative

This is normally used in mail to send different content-type

variants when the receiver's capabilities are not known. This is

not the case with HTTP.  Multipart/alternate may however be used to

provide meta information of many instances of an object, in the

case of a indirection response.  This allows, for example, pointers

to be returned by a name server to a set of instances of an object.

Multipart/related

This is the type to be used when the first body part contains

references to other parts which the server wishes to send at the

same time.  For example, the first part could be an HTML document,

and the included bodyparts could be the inline images mentioned

T. Berners-Lee                                                       21

within the text.

The body parts may have URI: fields if the body parts have URIs,

and so they may be referred to by these URIs in the body-parts.  If

the body-parts are transient (as in speech insertions in mail

messages) then the [propsed] "cid:" URI type may be used to refer

to them by content-identifier.

Multipart/mixed

This may be used to simply transfer an unrelated unstructured set

of objects.

Multipart/parallel

This may be used as in MIME to indicate simultaneous presentation.

[It is the author's belief that this is a trivial case of a

compound presentation which in general should be described by a

script which would be teh first bodypart of a multipart/related

document].

DATE: DATE

Creation date of object.  (or  last modified, and separately have a

Created: field?)  Format as in RFC850 but GMT MUST BE USED.

EXPIRES: DATE

Gives the date after which the information given ceases to be valid

and should be retrieved again. This allows control of caching

mechanisms, and also allows for the periodic refreshing of displays

of volatile data.  Format as for Date:. This does NOT imply that

the original object will cease to exist.

LAST-MODIFIED: DATE

Last time object was modified, i.e. the date of this version if the

document is a "living document". Format as for Date:.

MESSAGE-ID:  URI

A unique identifier for the message. As in RFC850 , except that the

unlimited lifetime of HTTP objects requires that the Message-ID be

unique in all time, not just in two years.

A document may only have one Message-ID.

No two documents, even if different versions of the same live

document, may have the same Message-id.

Note: Unlike the URI field, this does not fgive a way of accessing

the document, so the Message-Id cannot be used to refer to the

document. In the case of NNTP articles, the message-id may in fact

T. Berners-Lee                                                       22

be used within the URI for retrieval using NNTP.

URI:  1*URI

This gives a URI with which the object may be found.  There is no

guarantee that the object can be retrieved using the URI specified.

However, it is guaranteed that if an object is successfully

retrieved using that URI it will be to a certain given degree the

same object as this one.

If the URI is used to refer to a set of variants, then the

dimensiosn in which the variants may differ must be given with the

"vary" parameter:

Syntax          URI: <uri>  [ ; vary = dimension [ , dimension

]* ]

dimension       content-type | language | version

If no "vary" parameters are given, then the URI may not return

anything other than the same bit stream as this object.

Multiple occurencies of this field give alternative access names or

addresses for the object.

Examples

URI:  http://info.cern.ch/pub/www/doc/url6.multi; vary

=content-type

This indicates that retrieval given the URI will return the same

document, never an updated version, but optionally in a different

rendition.

URI:  http://info.cern.ch/pub/www/doc/url.multi;

vary=content-type, language, version

This indicates that the URI will return the smae document, possibly

in a different rendition, possibly updated, and without excluding

the provision of translations into different languages.URI:

http://info.cern.ch/pub/www/doc/url6.ps vary=content-typeThis

indicates that accessing the URI in question will return exactly

the same bitstream.

VERSION:

This is a string defining the version of an evolving object. Its

format is currently undefined , and so it should be treated as

opaque to the reader, defined by the informatiuon provider.  The

version field is used to indicate evolution along a single path of

a partucular work.  It is NOT used to indicate derived works (use a

link), translations , or renditions in different representations .

T. Berners-Lee                                                       23

Note: It would be useful to have sufficient  semantics to be able

to deduce whether one version predated or postdated another.

However, it may also be useful to be able to insert a particular

local code management system's own version stamp in this field.

Typically, publishers will have quite complex version information

containing hidden local semantics, giving value to the idea of this

field being opaque to other readers ofthe document.

DERIVED-FROM:

When an editied object is resubmitted using PUT for example, this

field gives the value of the Version . This typically allows a

server to check for example that two concurrent modifications by

different parties will not be lost, and for example to use

established version management techniques to merge both

modifications.

LANGUAGE: CODE

The language code is the ISO code for the language in which the

document is written.  If the language is not known, this field

should be omitted of course .

The language code is an ISO 3316 language code with an optional

ISO639 country code to specify a national variant.

Example

Language: en_UK

means that the content of the message is in British English, while

Language: en

means that the language is English in one of its forms. (@@ If a

document is in more than one language, for example requires both

Greek Latin and French to be understood, should this be

representable?)

See also: Accept-Language .

COST:  TBS

The cost of retrieving the object is given.  This is the cost of

access of a copyright work.  Format of units to be specified.

Currently refers to an unspecified charging scheme to be agreed out

of band between parties.

WWW-LINK:

Note. It is proposed that any HTML metainformation element (allowed

withing the HEAD as opposed to BODY element of the document) be a

valid candidate for an HTTP object header. WWW_Link is a required

T. Berners-Lee                                                       24

example. The suggestion was that the isomorphism should be realized

by prepending "WWW-" t the HTML element name to make the HTTP

header name, and the HTML attributes imply identically named

semicolon-separated MIME-style header parameters.  Other clear

candidates include WWW-Title.

It is open to discussion whether the "WWW-" should be removed.

This is semantically equivalent to the LINK element in an HTML

document which should be consulted for a full explanation.

Examples

WWW-Link:  href="http://info.cern.ch/a/b/c"; rel="includes"

WWW-Link:  href="mailto:timbl@info.cern.ch"; rev=made

The first example indicates that this object includes the specified

/a/b/c object.  The second indicates that the author of the object

is identified by the given mail address.

Note: Client tolerance of bad servers

Servers not implementing the specification as written are not HTTP

compiant.  Servers should always be made completely copmpliant.

However, clients should also tolerate deviant servers where

possible.

BACK COMPATIBILITY

In order that clients using the HTTP protocol should be able to

communicate with servers using the protocol originally implemented

in the W3 data model, clients should tolerate responses which do

not start with a numeric version number and response codes.

In this case, they should assume that the rest of the response is a

document body in type text/html.

WHITE SPACE

Clients should be tolerant in parsing response status lines, in

particular they should accept any sequence of white space (SP and

TAB) characters between fields.

Lines should be regarded as terminated by the Line Feed, and the

preceeding Carriage Return character ignored.

HTTP NEGOTIATION ALGORITM

This note defines the significance of the q, mxb and mxs values

optionally sent in the Accept: field of the HTTP protocol request

message.

It is assumed that there is a certain value of the presentation of

T. Berners-Lee                                                       25

the document, optimally rendered using all the information

available in its original source.

It is further assumed that one can allocate a number between 0 and

1 to represent the loss of value which occurs when a document is

rendered into a representation with loss of information.  Whilst

this is a very subjective measurement, and in fact largely a

function of the document in question, the approximation is made

that one can define this "degradation" figure as a function of

merely the representation involved.

The next assumption is that the other cost to the user of viewing

the document is a function of the time taken for presentation.  We

first assume that the cost is linear in time, and then assume that

the time is linear in the size of the message.

The final net value to the user can therefore be written

presented_value = initial_value * total-degradation  -   a  - b *

size

for a document in a given incoming representation.  Suppose we

normalize the initial value of the document to be 1.  The server

may judge that the value in a particular format is less than 1 is a

conversion on the server side has lost information. The total

degradation is then the product of any degradation due to

conversions internal to the server, and the degradation "q" sent in

the Accept field.   If q is not sent, it defaults to 1.

The values of a and b have components from processing time on the

server, network delays, and processing time on the client.  These

delays are not additive as a good system will pipeline the

processing, and whilst the result may be linear in message size,

calculation of it in advance is not simple.  The amount of

pipelining and the loads on machines and network are all difficult

to predict, so a very rough assumption must be made.

We  make the client responsible for taking into account network

delays. The client will in fact be in a better position to do this,

as the client will after one transaction be aware of the round-trip

time.

We assume that the delays imposed by the server and by the client

(including network) are additive.  We assume that the client's

delay is proportional to message size.

The three parameters given by the client to the server are

q                      The degradation (quality) factor between 0

and 1. If omitted, 1 is assumed.

mxb                    The size of message (in bytes) which even if

immediately available from the server will

T. Berners-Lee                                                       26

cause the value to the reader to become zero

mxs                    The delay (in seconds) which, even for a very

small message with no length-related penalty,

will cause the value to the reader to become

zero.

These parameters are chosen in part because they are easy to

visualize as the largest tolerable delay and size.  If not sent,

they default to infinity.

The server may optimize the presented value for the user when

deciding what to return.   The hope is that fine decisions will not

have to be made, as in most cases the results for different formats

will be very different, and there will be a clear winner.

A suitable algorithm is that the assumed value v of a document  of

initial value u delivered to the network after a delay t  whose

transfer length on the net is b bytes is

v  =  u * q   -   b/mxb  - t/mxs

Note that t is the time from the arrival of the request to the

first byte being available on the net.  [[See also: Design issues

discussions around this point.]]

HTTP Negotiation algoritm

This note defines the significance of the d, a and b values

optionally sent in the Accept: field of the HTTP protocol request

message.

It is assumed that there is a certain value of the presentation of

the document, optimally rendered using all the information

available in its original source.

It is further assumed that one can allocate a number between 0 and

1 to represent the loss of value which occurs when a document is

rendered intop a represenmtation with loss of information.  Whilst

this is a very subjective measurement, and in fact largely a

function of the document in question, the approximation is made

that one can define this "degradation" figure as a function of

merely the representation involved.

The next assumption is that there the other cost to the user of

viewing the doument is a function of the time taken for

presentation.  We first assume that the cost is linear in time, and

then assume that the time is linear in the size of the message.

The final net value to the user can therefore be written

presented_value = initial_value * total-degradation  -   a  - b *

size

T. Berners-Lee                                                       27

for a document in a given incoming represenattion.  Suppose we

normalize the initial value of the document to be 1.  The total

degradation is the product of any degradation due to conversions

internal to the server, and the degradation "d" sent in the Accept

field. The values of a and b are also sent by the protocol. If not

sent, they default to no cost (d=1, a=b=0).

The server may optimize the presented value for the user when

deciding what to return.   The hope is that fine decisions will not

have to be made, as in most cases the results for different formats

will be very different, and there will be a clear winner.

See also: Design issues discussions around this point.

Tim BL

Note: The cost of retrieval time

The assumption that the cost to the user associated with a certain

retrieval time is linear in that time is wildly innaccurate.  The

real function could be very dependent on circumstances (like go to

infinity at a deadline).

A better general approximation might be logarithmic for large time

delays, and linear for small ones, like a*log(b*t-1) which has two

parameters.

REGISTRATION AUTHORITY

The HTTP Registration Authority is responsible for maintaining

lists of:

Charge account name spaces (see ChargeTo: field above)

Authorization schemes (see Authorization: field above)

Data format names (as MIME Content-Types)

Data encoding names (as MIME Content-Encoding))

It is proposed that the Internet Assigned Numbers Authority or

their successors take this role.

Unregistered values may be used for experimental purposes if they

are start with "X-".

SECURITY CONSIDERATIONS

The HTTP protocol allows requests to communication to a remote

server machine, and all the expetant security considerations for

client-server systems apply, including

Authentication of requests

T. Berners-Lee                                                       28

Authenticationtion of servers

Privacy of request and response

Abuse of server features

Abuse of servers by exploiting server bugs

Unwitting actions on the net

Abuse of log information

The bulk of these are well known problems, tackled in part by some

featured of this protocol.  Some aspects particular to HTTP are

mentioned below.

Unwitting actions on the net

The writers of client software should be aware that the software

represents the user in his interactions over the net, and should be

careful to allow the user to be aware of any actions he may take

which may be taken as having an unexpected  significance by others.

TCP PORT NUMBERS

Clients should prompt a user before allowing HTTP access to

reserved ports other than the port resrverd for HTTP (port 80).

Otherwise, the user may unwittingly cause a transaction to occur in

some other (present or future) protocol.

IDEMPOTENT METHODS

The convention should be established that the GET and HEAD methods

never have the significance of taking an action.   The link "click

here to subscribe" causing the reading of a special "magic"

document is open to abuse by others making a link "click here to

see a pretty picture".  These methods should be considered "safe"

and should not have side effects. This allows the client software

to represent other, methods (such as POST) in a special way, so

that the use is aware of the fact that an action is being

requested.

Abuse of log information

A server is in the position to save large amount of personal data

about information requested by different readers.  This information

is clearly confidential in nature, and its handling may be

constrained by law in certain countries.  Server providers shall

ensure that such material is not distributed  without the

permission of any people or groups of people mentioned in the

results published.

T. Berners-Lee                                                       29

A feature which increases the amount of personal data transferred

is the Referer: field.  This allow reading patters to be studied,

reverse links drawn, and so is very useful.  Its power can be

abused of course if user details are not separated from the

Referee-Referer pairs.  Even when the personal information has been

removed, the Referer field may in fact be a secure document's URI,

whose revelation itself is breach of security. A method of

suppressing the Referer information in such cases may be the

subject of further study.

REFERENCES

RFC 822                 "Standard for ARPA Internet Text Messages".

David H. Crocker, describes Internet mail

message fromat.

RFC850                  "Standard for Interchange of USENET

Messages" This RFC uses some field names in

common with this specification, and is

relevant reading.

RFC977                  "Network News Transfer Protocol", Kantor and

Lampsley.

RFC 1341                Multipurpose Internet Mail Extensions

(MIME), Nathaniel Borenstien and Ned Freed,

Internet RFC 1341, 1992.  Now obsoleted by

RFC1521:

RFC 1521               MIME.  Not available in hypertext form yet.

RFC 1522               K Moore. "MIME: Part Two: Message Header

Extensions for Non-ASCII Text".

RFC1523                MIME text/enriched.

RFC 1524               MIME...

URL                     Universal Resource Locators. RFCxxx.

Currently available by anonymous FTP from

info.cern.ch as /pub/ietf/url3.{ps,txt}.

MIME and PEM            Internet Draft only

OBJECT CONTENTS

The data (if any) sent with an HTTP request or reply is in a format

and encoding defined by the object header fields, the default being

"plain/text" type with "8bit" encoding. Note that while all the

other information in the request (just as in the reply) is in ISO

Latin1 with lines delimited by Carriage Return/Line Feed pairs, the

data may contain 8-bit binary data.

T. Berners-Lee                                                       30

Termination

The delimiting of the message is determined by the Content-Length:

field. If this is present, then the message contains the specified

number of bytes.

Failing that, the content-type filed may contain a "bounday"

attribute which gives the boundary string with exacly the same

syntax as for a MIME multipart message.

Failing either of the above conditions, the data is terminated by

the closing of the connection by the sending party.  Note that this

method can not be used  for data sent with the request.

See also: note on server tolerance for back-compatibility, etc.

T. Berners-Lee                                                       31